=========================

3+OPEN INSTALLATION GUIDE

=========================

This guide instructs you on how to install the National Semiconductor

EtherNODE*16AT LAN Manager (NDIS) drivers for use with 3Com's 3+OPEN

LAN Manager Version 1.1 product.  These drivers may be used with

EtherNODE*16AT, 16ATX or 16AT-T boards. 3+OPEN does not have an

automated installation procedure for new hardware and drivers, so this

manual installation procedure is required.

1.  Install the EtherNODE adapter board in your PC as instructed

in the EtherNODE adapter manual.

2.  Install 3+OPEN LAN Manager on your server, OS/2 workstations, and DOS

workstations as instructed in the 3+OPEN "Installation and Setup Guide"

and Release Notes.  For each of these installations, select one of the

network adapters that is listed in the installation menu.  This will

be modified later.

3.  For the server, copy the \MSLANMAN.OS2\DRIVERS\ETHERNET\NE2000\NE2000.OS2

driver file to the \3OPEN\SERVER\LANMAN\DRIVERS directory.  You can

do this by running the COPY3SRV batch file from the \3OPEN directory

on the EtherNODE Utilities and Drivers diskette.  Enter

a:\3open\copy3srv

to do this.

For OS/2 workstations, copy the

\MSLANMAN.OS2\DRIVERS\ETHERNET\NE2000\NE2000.OS2 driver file to the

\3OPEN\OS2WKSTA\LANMAN\DRIVERS directory.  You can do this by running

the COPY3OS2 batch file from the \3OPEN directory on the EtherNODE

Utilities and Drivers diskette.  Enter

a:\3open\copy3os2

to do this.

For DOS workstations, copy the

\MSLANMAN.DOS\DRIVERS\ETHERNET\NE2000\NE2000.DOS driver file to the

\3OPEN\DOSWKSTA\LANMAN\DRIVERS directory.  You can do this by running

the COPY3DOS batch file from the \3OPEN directory on the EtherNODE

Utilities and Drivers diskette.  Enter

a:\3open\copy3dos

to do this.

NOTE:  All of these batch files assume that the EtherNODE Utilities

and Drivers diskette is in drive A: and that 3+OPEN is installed on

the current drive of your PC.  You may edit the batch files or type

them in manually if these drives are not correct for your configuration.

4.  For the server, copy the EtherNODE*16AT PROTOCOL.S22 file from the

\3OPEN directory of the EtherNODE Utilities and Drivers diskette:

copy a:\3open\protocol.s22 \3open\server\lanman\drivers

The \3OPEN\COPY3SRV batch file on the EtherNODE Utilities and Drivers

diskette does this at the same time as the copy in step 7 above.

For OS/2 workstations, copy the EtherNODE*16AT PROTOCOL.S22 file from

the \3OPEN directory of the EtherNODE Utilities and Drivers diskette:

copy a:\3open\protocol.s22 \3open\os2wksta\lanman\drivers

The \3OPEN\COPY3OS2 batch file on the EtherNODE Utilities and Drivers

diskette does this at the same time as the copy in step 7 above.

For DOS workstations, copy the EtherNODE*16AT PROTOCOL.DS2 file from

the \3OPEN directory of the EtherNODE Utilities and Drivers diskette:

copy a:\3open\protocol.ds2 \3open\doswksta\lanman\drivers

The \3OPEN\COPY3DOS batch file on the EtherNODE Utilities and Drivers

diskette does this at the same time as the copy in step 7 above.

5.  Edit the STARTUP.CMD file (for OS/2 workstations and the server) and

the AUTOEXEC.BAT file (for DOS workstations) to determine if they

contain NET USE commands which create drive "overlays" of installed

fixed disk drives on your PC.  If the file does contain NET USE

drive letters which conflict with your fixed disk drives, modify

the NET USE drive letters so that they do not conflict with your

system's fixed disk drive letters.  Save any changes that you make.

6.  Edit the CONFIG.SYS file to modify the selected hardware driver

from its current selection to NE2000.OS2 for the server and OS/2

workstations or to NE2000.DOS for DOS workstations.

For servers the modified line should read:

DEVICE = c:\3open\server\lanman\drivers\ne2000.os2

For OS/2 workstations the modified line should read:

DEVICE = c:\3open\os2wksta\lanman\drivers\ne2000.os2

For DOS workstations the modified line should read:

DEVICE = c:\3open\doswksta\lanman\drivers\ne2000.dos

Save any changes that you make.

7.  Edit the PROTOCOL.INI file in two places.

For servers the file name is

\3open\server\lanman\drivers\protocol.ini

For OS/2 workstations the file name is

\3open\os2wksta\lanman\drivers\protocol.ini

For DOS workstations the file name is

\3open\doswksta\lanman\drivers\protocol.ini

a.  Add this text near the beginning of the file (in the "ADAPTERS"

section):

[NSC2000_NIF]

; protocol.ini section for the National Semi. EtherNODE*16AT Card

IOBASE = 0x300

INTERRUPT = 3

DRIVERNAME = MS2000$

This text is available on the EtherNODE Utilities and Drivers

diskette in the file \3OPEN\PROTOCOL.S22 for OS/2 servers or

workstations or \3OPEN\PROTOCOL.DS2 for DOS workstations.

Modify the IOBASE and INTERRUPT parameters to match those of your

installed EtherNODE board.

b.  Change the BINDINGS entry in the [NBP] section of the "PROTOCOLS"

data entries near the end of this file from the selected hardware

to NSC2000_NIF.  The modified entry will look like:

BINDINGS = NSC2000_NIF

Save any changes that you make.

8.  Reboot the system for the changes to take effect.

Trademarks:

3Com is a registered trademark of 3Com Corporation.

3+OPEN is a trademark of 3Com Corporation.

LAN Manager is a trademark of Microsoft Corporation.

OS/2 is a trademark of International Business Machines Corporation.