#

| README file for

|

| ELSA tool SATURN.EXE, ver. 1.0

|

| Copyright (c) 1993-97 ELSA AG, Aachen (Germany)

|

|                                     Subsidary:

| ELSA AG                           ELSA Inc.

| Sonnenweg 11                        2150 Trade Zone Blvd.

| D-52070 Aachen                      Suite 101

| Germany                             San Jose, CA 95131

|                                     USA

|

| Phone      : +49-241-606-0          Phone: +1-408-935-0350

| Support Fax: +49-241-606-6399       Phone:  1-800-272-ELSA

| BBS (modem): +49-241-606-9800       Fax  : +1-408-935-0370

| BBS (ISDN) : +49-241-606-9820       BBS  : +1-408-935-0380

| CompuServe : GO ELSA                CIS  : GO ELSA

| WWW        : http://www.elsa.de     WWW  : http://www.elsa.com

|

| 04/23/97, gu, MGroegor

Table of Contents:

1.      Description

2.      Usage

3.      Hints

------------------------------------------------------------------------

1.      Description

The intel SATURN chip set is most popular with personal

computers featuring the i486 microprocessor and the PCI bus. On

many of these machines, however, the chip set is not

initialized in the optimum way by the system BIOS.

To provide the full graphics power of the ELSA WINNER graphics

accelerator boards on the PCI bus in computers with the intel

SATURN chip set, the chip set has to be reconfigured.

The SATURN.EXE program is provided to allow this reconfigura-

tion. When used with option "BON" (for "burst on"), the SATURN

chip set will be set to use burst mode access with minimum wait

states on the PCI bus.

Extensive tests have shown the potential to dramatically

increase graphics performance of the WINNER PCI boards using

this tool.

There have been no reports of problems or incompatibilities

with other PCI cards. However, the SATURN.EXE program may be

called with option "BOFF" (for "burst off"), should any problems

arise.  This will turn off the burst access mode on the PCI bus.

The SATURN.EXE program can be used with any ELSA WINNER

graphics accelerator board on the PCI bus.

!!! ATTENTION !!!

The WINNER 1000PCI is a special case; due to an error in the

design of the S3 86C928P chip, the SATURN.EXE program is

necessary to use the WINNER 1000PCI in a computer with the

intel SATURN chip set. It is recommended to use the "BON"

option to achieve maximum performance.

------------------------------------------------------------------------

2.      Usage

The SATURN.EXE program is provided to turn on and off the PCI

burst mode of the intel SATURN chip set.

Usage:

SATURN  BON           - turn on burst mode for the PCI bus

SATURN  BOFF          - turn off burst mode for the PCI bus

SATURN                - show short usage info

------------------------------------------------------------------------

3.      Hints

The SATURN.EXE program should be included in the AUTOEXEC.BAT

file of your computer to automatically turn on or off the PCI

burst mode every time the computer starts running.

On systems with the intel SATURN chip set that provide a "Quick

BOOT" option in the setup menu, this option should be enabled.