NT40ND4.DOC

Kingston Technology Company

INSTALLATION GUIDE

Microsoft Windows NT Ver.4.0 NDIS 4 Driver

for

EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Copyright (C) 1996 Kingston Technology Company.

All rights reserved.

This installation guide provides the following instructions:

-  Installing the Microsoft Windows NT NDIS 4 driver.

Driver Installation Procedure:

This installation procedure assumes your Windows NT Ver.4.0 software

is properly installed on your computer.

I. Installing Windows NT 4.0 NDIS 4 Driver

1.

Make sure the EtheRx PCI adapter is properly installed in

your computer.

2.

Logon as the Administrator.Open "My Computer", then "Control Panel".

Double click on "Network"

3.

In the "Adapters" tab, choose 'Add'.

4.

From the "Select Network Adapter" window, select 'Have Disk'

5.

Insert "Kingston Drivers diskette", make sure the directory

path is "A:\NDIS4" and click OK to continue.

6.

Highlight "Kingston EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)"

from the "Select OEM option" window and click OK to continue.

7.

In the "Kingston EtheRx PCI 10/100 Fast Ethernet Adapter (KNE110TX)

Setup" window, click OK to select "AutoDetect" connection type.

8.

You can then select supported services and protocol in the

"Services" tab and 'Protocol' tab.

9.

After you have made your choices, close the "Network" window.

Restart your computer in order for the changes to take effect.

10.

The installation is now complete.

Kingston Technology, EtheRx, and QStart are registered

trademarks of Kingston Technology Company.  All others are

trademarks or registered trademarks of their respective holders.