*****************************************************

*             ATAPI CD-ROM drive                    *

*****************************************************

/////////////////////////////////////////////////////

///                                               ///

///  Manual for using installer (DOS, OS/2)       ///

///                                               ///

///             Making   date:  2/20/95           ///

///             Modified date:  5/15/95           ///

///             Modified date:  7/31/95           ///

///             Modified date:  9/27/95           ///

///             Written by :  MKE(Panasonic)      ///

/////////////////////////////////////////////////////

*******************************************************************************

(A)	PC-DOS and MS-DOS installtion Procedure

(also supported for Windows 3.1 / Windows For Workgroup 3.11)

*******************************************************************************

1. Supportted CD-ROM drive:

<ATAPI Interface CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

CR-581-x (quadruple speed, tray loading type)

2. Installer file name:

INSTALL.EXE (Version 1.10, common file for installation)

INSTALL.DAT (installation's data file for ATAPI CD-ROM)

3. Discription for execution:

Please type following from the DOS prompt:

'A:'[return]  or 'B:'[return]

'INSTALL' [return]

*******************************************************************************

(B)	OS/2 2.1, 2.11, and Warp 3.0 installtion Procedure

*******************************************************************************

1. Supportted CD-ROM drive:

<ATAPI Interface CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

CR-581-x (quadruple speed, tray loading type)

2. Installer file name:

INSTOS2.EXE (Version 2.10, common file for installation)

INSTOS2.DAT (installation's data file for ATAPI CD-ROM)

3. Discription for execution:

Please type following from the DOS prompt:

'A:'[return]  or 'B:'[return]

'INSTOS2' [return]

Trademarks and Copyrights:

PC-DOS is a trademarks of International Business Machines Corporation.

MS-DOS is a trademark of Microsoft corporation.

OS/2 is a trademarks of International Business Machines Corporation.

Windows 3.1 is a trademark of Microsoft corporation.

Sound Blaster is a trademark of Creative Labs, Inc.

------------ cut here ------------ cut here ------------ cut here ------------

/////////////////////////////////////////////////////

///                                               ///

///Device Driver supportted STATUS in CD-ROM drive///

///                                               ///

///             Making   date:  3/ 3/95           ///

///             Modified date:  5/16/95           ///

///             Modified date:  7/31/95           ///

///             Modified date:  9/20/95           ///

///             Written by :  MKE(Panasonic)      ///

/////////////////////////////////////////////////////

*******************************************************************************

(C)	PC-DOS and MS-DOS device driver support

(also supported for Windows 3.1 / Windows For Workgroup3.11)

*******************************************************************************

1. Supportted CD-ROM drive:

<ATAPI Interface CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

CR-581-x (quadruple speed, tray loading type)

2. Device Driver name:

CR_ATAPI.SYS (Version 5.17 ) for DOS

WDCTRL.386   (Version 1.10 ) for 32bitDiskAccess=ON in Windows3.1

!!! CAUTION !!!

You can not use WDCTRL.386, when you use 32bitFileAccess=ON in WFW3.11.

Please change 32bitFileAccess=ON to 32bitFileAccess=OFF.

3. Driver discription:

[/D:xxxxxxx]	---> Device name option

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000

*[/P:nnn]	---> ATAPI base I/O Port Address(hex) option

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /P:nnn

nnn==1F0, 170, 1E8 or 168

Primary connection:	nnn==1F0

Secondary connection:	nnn==170

Tertiary connection:	nnn==1E8(Sound Blaster IDE)

Quaternary connection:	nnn==168(Sound Blaster IDE)

*[/I:xx]	---> Interrupt number option(IRQ=10,11,12,14,15)

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /I:xx

Primary connection:	xx==14,10,11,12 or 15

Secondary connection:	xx==15,10,11,12 or 14

Tertiary connection:	xx==10,11,12,14 or 15

Quaternary connection:	xx==10,11,12,14 or 15

*[/W:n]		---> Waiting value option

When PC read data from CD-ROM drive, Driver insert waiting

value into read cycle one by one.

If you are using PC that are not supported IOCHRDY, please set

this value(waiting value).

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /W:n

n==0, 1, 2, ... 255

(For example 106usec/1 wait in 486DX2 66MHz)

*[/Q]		---> Non stop option

Non stop(no message <A>bort, <R>etry) in initializing driver.

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /Q

*[/E]		---> Available error correction

This option is available error correction for CD-G application.

(Player for not including error correction)

ex.)DEVICE=CR_ATAPI.SYS /D:MSCD000 /E

*[/B]		---> No reset to CD-ROM Drive at boot configration

This option make invalid that Device Driver reset to CD-ROM

drive at boot sequence(in CONFIG.SYS).

]

* : You can omit this option.

*******************************************************************************

(D)	OS/2 2.1, 2.11, and Warp 3.0 device driver support

*******************************************************************************

1. Supportted CD-ROM drive:

<ATAPI Interface CD-ROM>

CR-572-B (double speed, tray loading type)

CR-574-B (quadruple speed, tray loading type)

CR-581-x (quadruple speed, tray loading type)

2. Device Driver name:

MKEATAPI.ADD (Version 1.12)

3. Driver discription:

*[/V]		---> Verbose Mode option

Displays controller status and drive type during OS/2

initialization.

*[/A:n]		---> Adapter Number option

Adapter 0 has default parameters for the primary adapter.

Adapter 1 has default parameters for the secondary adapter.

*[/P:nnn]	---> ATAPI base I/O Port Address(hex) option

This parameter overrides the default base I/O port address for

the adapter indecated. The default I/O address for primary is

(1F0) and for seconday is (170).

If you will set this parameter, you should set /I: parameter.

*[/I:xx]	---> Interrupt number option

This parameter overrides the default IRQ Number for the adapter

indecated.The default IRQ level for primary is (14) and for

secondary is (15).If you will set this parameter, you should

set /P: parameter.

*[/NI]		---> No Interrupt Mode( Polling Mode ) option

When you use IDE HDD and ATAPI CD-ROM drive at the same adapter

(primary or secondary), the MKEATAPI.ADD driver is automatically

used this mode.

*[/IOR]		---> I/O Channel Ready Supported Mode option

For the adapter with I/O channel ready, the MKEATAPI.ADD driver

can designate the fastest read mode.

* : You can omit this option.

Samples:

; scan to primary and secondary adapter.

BASEDEV=MKEATAPI.ADD /V

; scan to secondary adapter with I/O channel ready.

BASEDEV=MKEATAPI.ADD /V /A:1 /IOR

; scan to another adapter(I/O address 1E8  IRQ 11) with I/O channel ready.

BASEDEV=MKEATAPI.ADD /V /P:1E8 /I:11 /IOR

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

CAUTION:

OS/2 Warp has ATAPI CD-ROM filter driver(IBMIDECD.FLT). If problems occur

using the IBMIDECD filter, you fix lines in your OS/2 CONFIG.SYS file has

to be edited as follows:

rem BASEDEV=IBMIDECD.FLT

At the end of the file, add the following statement

BASEDEV=MKEATAPI.ADD /V

and copy to root directory on the OS/2 system drive.

In this time, if the CR-ROM drive is attached as master to primary or

secondary, you should fix a line in your OS/2 CONFIG.SYS file has to be

edited as follows:

BASEDEV=IBM1S506.ADD /A:x /I

The /A parameter refers to the adapter number that the CD-ROM is attached

to.The /I parameter indicates that the IBM1S506 driver should not attempt

to initialize the adapter indicated.

!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!!

Trademarks and Copyrights:

PC-DOS is a trademarks of International Business Machines Corporation.

MS-DOS is a trademark of Microsoft corporation.

OS/2 is a trademarks of International Business Machines Corporation.

Windows 3.1 is a trademark of Microsoft corporation.

Sound Blaster is a trademark of Creative Labs, Inc.