DLG Viewer 2.00 - USGS DLG Viewer and GPS Moving Map Display

------------------------------------------------------------

DLG Viewer is a United States Geological Service (USGS)

Digital Line Graph (DLG) viewing and GPS moving map display

utility.  DLG is a DOS program ideally suited to run on the HP

200LX Palmtop computer.  Since it is a plain DOS program, it

will run on any DOS compatible computer.  However, the program

supports only the CGA display.

Features:

---------

o   reads USGS 100k optional format DLG.

o   zooms in a window within the area represented in the

DLG file.

o   mosaic several DLG files into a single map image

o   superimposes several DLG files in a single map image

o   NMEA interface to GPS for moving map display

o   automatic switching to neighbouring maps

o   logs tracks made from GPS co-ordinate

o   displays recorded log

o   read track recorded by Garmin's 12XL GPS

DLG reads the Optional Format 100k USGS DLG.  It can also store

the DLG map in compressed bit map format (DLB.)  The 100k DLG

maps may be downloaded from:

ftp://edcftp.cr.usgs.gov/pub/data/DLG

The DLG Data User Guide is at:

ftp://mapping.usgs.gov/pub/ti/DLG/100kdlgguide

Legal Notices

-------------

In the following paragraphs, the terms 'the software,' 'this

software,' and 'it' refer to DLG Viewer.  The author refers to

me, Alfred Lee.  I may be contacted at the Internet address

'alfred1520@geocities.com'.  In the event that I am not

reachable at this address, please conduct a search on the any

popular Internet search engine for 'alfred1520'.

Copyrights

----------

This software is copyrighted Freeware.  World wide copyrights

are reserved by the author.

The author also reserves the rights to re-issue the software

as Shareware or as commercial software.  The Freeware status

of this software shall terminate at such time.  You are then

requested to acquire an appropriate license to continue using

the software.

Distribution

------------

Since this software is currently being released as Freeware,

you may re-distribute this software subjecting to each and

all of the following conditions:

1.  This document must be present.

2.  All files are distributed together as a single package.

3.  All files remain in their original form without any

modification.

4.  The re-distribution will result in absolutely no direct

or indirect financial gains for all of the parties

involved.

5.  The re-distribution is non-commercial.

The followings are strictly prohibited without explicit

written permission from the author:

1.  Inclusion of any part or the whole of this program in any

periodical, book, or any other publication, or bundling

with any other package or product, or adding it to any

software collection for any direct or indirect commercial

gains or any other commercial purposes or promotions.

Distribution with no possibility of direct or indirect

financial gain is permitted.

2.  Inclusion of any part or the whole of this program on any

Internet Wed site if such a site is commercially oriented

and if any financial gain can be derived from such an

inclusion.  Distribution with no possibility of direct or

indirect financial gains is permitted.

3.  Charging a fee for re-distribution.

4.  Implying or referencing to this software in an attempt to

enhance the commercial value of any commercial product for

financial gain or commercial promotion.

5.  Using this software in an educational or teaching

institution that charges a fee or tuition.  Non-profit

institutions that do not charge a fee nor tuition or only

charge a minimal material fee are exempted.

Any exception requires the explicit written permission form

the author and perhaps compensation to the author.

Warranty

--------

This software is provided as is without any warranty,

expressed or implied.  This includes without limitation the

fitness of purpose or application.

This software has not been extensively tested.  It has only

been tested by a single user (the author) on a particular

computer (HP200LX.)  It is never implied that the program will

function properly and harmlessly on a different machine due to

interaction between this program and any and all other

installed components on your particular computer.

The author shall not be liable for any damages, including any

lost profits, whether direct, indirect, special, incidental,

or consequential arising from a failure of this program or

accompanying files to operate in a manner desired by the user.

The author shall not be liable for any damage to data or

property which may be caused directly or indirectly by the use

of this program.  You shall bear the all the risks that may

arrise out of the use of this software.

This software may use unproven software routines to handle

the serial port communication.  Please use the documented

method to exit the software.  Failure to following this

instruction will result in the Interrupt Vector not being

properly restored and causes the system to lock up and may

result in file system corruption or loss of data.  Backup you

data.

List of files:

--------------

SALNVY08.DLG    sample DLG map

BIGBEAR.DLB     sample DLB mapp

DEATHVLY.TRK    sample track in Death Valley

EXCLUDE.EXL     sample control file to exclude features in DLG

DLG.DOC         this file

DLG.EXE         the executable

LINESTYL.STY    sample control for line style

USCON.PCX       sample PCX map in sinusoidal projection

USCON.MAP       scale information for sinusoidal projection

Quick start:

------------

Demo 1: loading DLG and saving in propriotary binary format

1.  dlg -h640 newbry02.dlg<Enter>           // at C:\DLG\>

2.  <Esc>                                   // at prompt

3.  s                             // after map is displayed

4.  newbry02.dlb                  // save in .dlb format

5.  <arrow keys>, <Pg Up/Down>, <Home>, <End>   // move

6.  ctrl^<Pg Up/Down>, ctrl^<Home>, ctrl^<End>  // scroll

7.  q                                           // to quit

Demo 2:

1.  dlg newbry02.dlb

2.  x                                 // generate position

3.  t                                 // trace track

4.  q                                 // to quit

PCX maps in rectangular projection:

-----------------------------------

DLG Viewer reads rectangular projection (all longitudes are

horizontal and all latitudes are vertical) maps store in PCX

format.  An auxiliary file with the same filename and the

extension '.map' is required to supply the co-ordinate

information.  The auxiliary file has the following format:

0 0 0 0

0 0 50.47482 124.432

1599 949 23.28759 64.6826

Line #1:

--------

(four zeros separated by spaces for compatibility reason)

Line #2:

--------

(image co-ordinate and actual longitude and latitude of a

point on the map)

X1              x pixel co-ordinate (left is 0)

Y1              y pixel co-ordinate (top is 0)

Lat1            latitude (north is positive)

Lon1            longitude (west is positive)

Line #3:

--------

(image co-ordinate and actual longitude and latidute of

another point on the map)

X2              x pixel co-ordinate (left is 0)

Y2              y pixel co-ordinate (top is 0)

Lat2            latitude (north is positive)

Lon2            longitude (west is positive)

Notes: all numbers are separated by one or more spaces.

Generating PCX maps in sinusoidal projection:

---------------------------------------------

The following server allows the generated image size to be

specified:

http://tiger.census.gov/cgi-bin/mapbrowser

The generated file is in GIF format.  You must convert it to

1-bit PCX format.

PCX maps in sinusoidal projection:

----------------------------------

DLG Viewer reads sinusoidal projection (map projection used

on the TIGER server) maps store in PCX format.  An auxiliary

file with the same filename and the extension '.map' is

required to supply the co-ordinate information.  In sinusoidal

projection the scales are true along the centre meridian and

all parallels.  DLG needs the x/y and long/lat co-ordinates of

two specific points to compute the long/lat any where on the

map.  The first point is a point on the centre parallel west

(preferable close to the left edge) of the centre meridian.

The second point is a point on the centre meridian north of

the centre parallel (preferable close to the top edge.)  The

auxiliary file has the following format:

0 0 0 0

117 500 35.0 120.0

803 167 45.0  95.0

1

Line #1:

--------

(four zeros separated by spaces for compatibility reason)

Line #2:

--------

(image co-ordinate and actual longitude and latitude of a

point on the map)

X1              x pixel co-ordinate (left is 0)

Y1              y pixel co-ordinate (top is 0)

Lat1            latitude (north is positive)

Lon1            longitude (west is positive)

Line #3:

--------

(image co-ordinate and actual longitude and latidute of

another point on the map)

X2              x pixel co-ordinate (left is 0)

Y2              y pixel co-ordinate (top is 0)

Lat2            latitude (north is positive)

Lon2            longitude (west is positive)

Line #4:

--------

1               specifies sinusoidal projection

Notes: all numbers are separated by one or more spaces.

Command line argument:

----------------------

dlg [options] <filename.ext>

-a<fname> : list of attributes to exclude

This is a text file listing major/minor attribute code

pairs to be excluded., e.g.

170  210

170  211

170  212

-b<#>  : background

-d<fname> : list of attributes and line style

This is a text file listing major/minor attribute code

pairs, run, and dash.  Run and dash are two integers

that control the line style.  Dash is the number of

dots.  (Run - dash) is the number of spaces.  Examples

0    0      0 0

170  210    0 0

170  211    2 1

170  212    4 2

The first line is the default line style.

-h<d>  : height, 200 <= d

These options defines the size of the image for

display.  The minimium size is 640 wide by 200

high.  The maximium size is limited by DOS and

is typically 1600 x 1600.

-g     : create dlg.log

-l<#>/<filename> : record GPS co-ordinate

# is recording interval in seconds. 'filename' is

output filename; appends to existing files.

-m<#>  : create #^2 maps mosaic

-m<#>:<#2>  : create # high by #2 wide maps mosaic

Creates a square mosiac of the given size, e.g.

-m4 creates a 4 x 4

-m2:4 creates a 2 high x 4 wide

-Mx<#> : offset 1st mosaic # to right

-My<#> : offset 1st mosaic # down

-n     : enable NMEA

-p<#>  : palette

-w<d>  : width,  640 <= d

-xp<f> : x  pan, 0 < f < 1

-xz<f> : x zoom, 1 < f

-yp<f> : y  pan, 0 < f < 1

-yz<f> : y zoom, 1 < f

These options defines a window into the the DLG being

read.  All <f>'s are real numbers.

X and y zooms are the magnification factors.  The

default values are 1.0 which cover the entire area

defined in the DLG map.  2.0 for both x and y zoom

would cover one quarter the area of the DLG map.

X and y pan are the offset from the top left corner.

These values must be in the range between 0.0 and 1.0.

The default values are 0.0 which indicate no offset.

In the following example a window is made in the lower

right quarter:

-xp0.5 -yp0.5 -xz2.0 -yz2.0

Keyboard commands:

------------------

' '

Print status.

1

Toggle between sving dlb maps in compressed format for

space (default) or saving in uncompressed format for

speed.  <F1> to see current settings.

a

Annotation.

b

Erases map and prompts new corner co-ordinates to

reset map scale.  Enter 0 for co-ordinates for no

re-scale.

c

Enable/disable confining cursor in centre of screen.

C

Scroll current cursor position to the centre of the

screen.

d

Draw a circle centre s on the cursor.

D

Change radius for command 'd': 0.5 km, 1km, 2km, 5 km,

10 km.

f

Help find cursor.  Leaves a heavy trail.  Press 'f'

again to exit.

g

Superimpose 3.75 minute grids on screen.

G

Scale to 7.5 minutes paper map grid.  DeLorme Atlas

includes longitude and latitude tick marks at 7.5

minutes interval.  Use the left and the right arrow

keys and the '4' and '6' keys to measure the physical

length of 7.5 minutes on the paper map.  Once the

dimensions are measure, the cross-hair reads the

approximate co-ordinates of the paper map.  Press the

'Enter' key will transfer the co-ordinate to the map.

i

Inverts map

k

This key is modal.

If no track file has been opened, a filename for the

GPS track and a delay tick is prompted.  The track

file should have the file format generated by Garmin's

12LX GPS.  The delay tick is 0 or larger.  This value

is the time delay between each track point.  Each

delay tick is 1/18 of a second.

If a track file has been opened, this key toggles

between free running and stepping track points.

K

If a track file has been opened, this key back step

a step and enter step mode.

^k

While single stepping a track file, this key single

step forward.

l

Enable/disable GPS logging.  Enter filename and

recording recording interval when enabling.

L   play GPS back

Enter GPS log play back mode.  Enter filename when

Activating.  The following commands are available:

<F2>

|<<

Rewind to the start of the file and enter step

mode.

<F3>

<<<

Fast rewind to the last stop.

<F4>

<<

Run backward.

<F5>

<

Step backward.

<F6>

>

Step forward.

<F7>

>>

Run forward.

<F8>

>>>

Fast forward to the next stop.

<F9>

goto

Goto a specified point offset from start.

m

Cycle through zoom ratio of 1, 2, and 4.

M

Load DLG map.

n

Enable/disable GPS NMEA message reception on com1 <F1>

to see current settings.

p

Save map in .bmp format.

P

Save map in .pcx format.

q

Quit DLG.

r

Display readouts on the left/right of the screen.

s

Save map in binary dlb format.  See related command: 1

t

Enable/disable GPS track plotting.  <F1> tosee current

settings.

T

Enter text on map.

u

zoom out.

w

Write waypoints to waypoint file.  A filename will be

prompted when press for the first time.  Garmin GPS

format will be used.  Append a '#' will cause all

subsequent name to use the same prefix follow by a

three digits sequence number starting from 001.

W

Write track to track file.  A filename will be

prompted when press for the first time.  Garmin GPS

format will be used.  The track file can be uploaded

in to the Garmin.

x

Simulate GPS message reception.

<F1>

Help screen and current settings.

<arrows>

Move cursor.

shift-<arrows>

Move cursor in large steps.

Fn-<arrows>

Scroll map.

ctrl-Fn-<arrows>

Scroll map in large steps.

<tab>

pause"

<Enter>

While single stepping a track file, this key fast

forward to the next stop.  A stop is GPS record

interval of greater than 60 seconds.

Major and minro attribute codes:

--------------------------------

170 0201 Primary route, class 1, symbol undivided

0202 Primary route, class 1, symbol divided by centerline

0203 Primary route, class 1, divided, lanes separated

0204 Primary route, class 1, one way, other than divided

highway

0205 Secondary route, class 2, symbol undivided

0206 Secondary route, class 2, symbol divided by centerline

0207 Secondary route, class 2, symbol divided, lanes

separated

0208 Secondary route, class 2, one way, other then

divided highway

0209 Road or street, class 3

0210 Road or street, class 4

0211 Trail, class 5, other than four-wheel drive vehicle

0212 Trail, class 5, four-wheel-drive vehicle

0213 Footbridge

0214 Ferry crossing

0215 Perimeter of parking area

0216 Arbitrary extension of line (join or closure)

0217 Road or street, class 3, symbol divided by centerline

0218 Road or street, class 3, divided lanes separated

0221 Road in street, class 3, one way

0222 Road in transition

170 0401 Traffic circle

0402 Cloverleaf or interchange

0403 Toll gate, toll plaza or perimeter or toll plaza

0404 Weigh station

0405 Nonstandard section of road

170 0600 Historical

0601 In tunnel

0602 Overpassing, on bridge

0603 Under construction, classification known

0604 Under construction, classification unknown

0605 Labeled "old railroad grade"

0606 Submerged or in ford

0607 Underpassing

*0608 Limited access

0609 Toll road

0610 Privately operated or controlled public access

0611 Proposed

0612 Double-decked

0613 In service facility or rest area

0614 Elevated

0615 Bypass route

0616 Alternate route

0617 Business route

0618 On drawbridge

0619 Spur

0620 Loop

0621 Connector

0622 Truck route

0650 Road width 46-55 feet, 0.025 inches at 1:24,000

0651 Road width 56-65 feet, 0.030 inches at 1:24,000

0652 Road width 66-75 feet, 0.035 inches at 1:24,000

0653 Road width 76-85 feet, 0.040 inches at 1:24,000

0654 Road width 86-95 feet, 0.045 inches at 1:24,000

0655 Road width 96-105 feet, 0.050 inches at 1:24,000

0656 Road width 106-115 feet, 0.055 inches at 1:24,000

0657 Road width 116-125 feet, 0.060 inches at 1:24,000

0658 Road width 126-135 feet, 0.065 inches at 1:24,000

0659 Road width 136-145 feet, 0.070 inches at 1:24,000

170 0000 Photorevised feature

171 ---- Number of lanes

172 ---- Interstate route number

173 ---- U.S. route number

174 ---- State route number

175 ---- Reservation, park, or military route number

176 ---- County route

177 XXYY Alphabetic portion of any route

number.  Substitute numeric equiva-

lent of alphabetic for XX and for YY

as follows:  00 = blank, 01 = A,

02 = B, 03 = C, 04 = D, 05 = E,

06 = F, 07 = G, 08 = H, 09 = I,

10 = J, 11 = K, 12 = L, 13 = M,

14 = N, 15 = O, 16 = P, 17 = Q,

18 = R, 19 = S, 20 = T, 21 = U,

22 = V, 23 = W, 24 = X, 25 = Y,

26 = Z.

178 0000 Best estimate of position or classification

179 00-- Coincident feature

-------------------------

If you would like to see this software being maintained,

please me a note indicating the name of the software and that

you are a regular user of the software.  I will base my time

allocation for software development on the number of

responses I receive.

Future updates, if any, will be posted at:

http://www.geocities.com/CapeCanaveral/Hangar/9412

Please send comments, suggestions, bug reports to

alfred1520@geocities.com

(if this address becomes obsolete, please search for

alfred1520 on Internet search engines.)

so that I can consider them for a future release.

Alfred Lee, Aug 4, 1998

Change History:

---------------

98/03/14 -  Version 1.0, the original release

98/03/28 -  Version 1.01, minor fixes:

o   reads UTM zone number from DLG maps

o   correctly accepts dlg a:*.dlg

98/05/06 -  Version 1.02, new features:

o   creates waypoints and track files for

downloading to Garmin 12XL

98/05/13 -  Version 1.10, bug fix:

o   change to use CLARKE 1866 ellipsoid for UTM/

long/lat conversion.  This corresponds to

the DLG's NAD27 CONUS datum

o   non-square moosaic is now possible

98/07/10 -  Version 1.20, new features:

o   erase and re-scale map

o   enable NMEA from command line

o   enable GPS recording from command line

o   superimpose 3.75 minutes frid on screen

o   paper map scaler

98/08/04 -  Version 2.00, new features:

o   reads rectangular projection PCX maps

o   reads sinusoidal projection PCX maps

o   invert map