New  DISKTEST  documentation - changes as of 07.04.2000

-----------Command-----------    -------------------Output-------------------

DISKTEST                         Usage parameters.

DISKTEST  testfilesize           Create a test file of 'testfilesize' size,

then perform a "random access" test of the

disk drive's "seek time".

The number of accesses is 5000, and the

size of each buffer loaded is 2048 bytes.

For worst-case seek times of approximately

30 milliseconds, this process could take

up to 15 minutes to complete (allowing

several minutes to write the test file).

While seek times as low as 5 milliseconds

are often quoted for modern drives, those

times are actually dependent on cacheing

software built into the drive, or a cache

which is part of the operating system.

The caches just mentioned are quite useful

for casual operation of the computer, or

even heavy use by most persons who do not

depend on continuous access to large local

or local-network databases.

In the latter cases, where a computer's RAM

memory is, say, 128 mb or so, and a database

of one to several gigabytes is being heavily

accessed for updating files, printing reports

and so on, those caches can become full very

quickly, and thus the disk drive is relegated

to "no cache" status effectively, where seek

times average on the order of 25 milliseconds.

To get an idea of what your disk drive's

actual hardware seek time performance is,

I recommmend:

For Windows 9x users, first try specifying

a 'testfilesize' of 2047 (2047 mb, if that

much disk space is available).  The ideal

test file size should be a minimum of 5 to

10 times the size of the RAM memory of the

computer, or, if that much disk space isn't

available or RAM memory is 1 gigabyte or more,

Windows 9x users can shut down and reboot to

the "Command Prompt Only" menu selection and

proceed with a smaller file size. This reboot

assumes that the "Command Prompt Only" mode

(DOS) does not load a cache program such as

SMARTDRV.EXE.

Windows NT, Windows 2000, and non-Windows

users can use this utility effectively if

they have at least 5 times the amount of

free disk space as they have RAM memory,

within the maximum 'testfilesize' limit

of 2047 mb (i.e., approx. 2 gigabytes).