Specific Uses for HOOKINT

-------------------------

Intel SatisFAXtion Board

------------------------

Without the file HOOKINT.COM, DESQview users often find that they cannot

successfully receive a FAX when the Intel SatisFAXtion software is running in

background and multitasking is going on.  For the most effective configuration

of the SatisFAXtion software in DESQview:

1) Load SATISFAX.SYS in the CONFIG.SYS file.

2) Determine the hardware interrupt that the SatisFAXtion board uses for FAXes.

To do so, run the SatisFAXtion SETUP.EXE program, go to the Advanced Setup,

select Set Up Hardware, and look at the current configuration.  The currently

shipping version of the SatisFAXtion board puts the FAX on IRQ10 by default, but

offers seven different options.  Here is a list of the seven different IRQ's

that Intel lets you select from, with the corresponding INT numbers.

IRQ            INT

---            ---

2              A

3              B

4              C

5              D

7              F

10             72

11             73

If you determine that the FAX is using IRQ10, then 72 is the INT number that

HOOKINT.COM will use as its first parameter.

3) Load a batch file in a DV window that contains the following lines:

HOOKINT 72 2F

CASMGR CASMGR.CFG

FAXPOP

FAX

If the FAX is not using IRQ10, substitute another number from the above table

instead of the 72 on the HOOKINT line.

Use a DESQview window with the following Change a Program options set:

Program Name............: Intel SatisFAXtion

Keys to Use on Open Menu: SA                         Memory Size (in K): 480

----------------------------------------------------------------------------

Program...: (name_of_your_batch_file)

Parameters:

Directory.: (name_of_SatisFAXtion_directory)

----------------------------------------------------------------------------

Options:

Writes text directly to screen.......: [Y]

Displays graphics information........: [Y]

Virtualize text/graphics (Y,N,T).....: [Y]

Uses serial ports (Y,N,1,2)..........: [Y]

Requires floppy diskette.............: [N]

System Memory (in K).......:   0   Maximum Program Memory Size (in K)..:

Script Buffer Size.......:  1000   Maximum Expanded Memory Size (in K):

Text Pages: 1  Graphics Pages: 1   Initial Mode:        Interrupts: 00 to FF

----------------------------------------------------------------------------

Window Position:

Maximum Height:  25       Starting Height:           Starting Row...:

Maximum Width.:  80       Starting Width.:           Starting Column:

----------------------------------------------------------------------------

Shared Program

Pathname..:

Data......:

----------------------------------------------------------------------------

Close on exit (Y,N,blank)......: [ ]  Uses its own colors..............: [Y]

Allow Close Window command.....: [N]  Runs in background (Y,N,blank)...: [ ]

Uses math coprocessor..........: [Y]  Keyboard conflict (0-F)..........: [0]

Share CPU when foreground......: [Y]  Share EGA when foreground/zoomed.: [Y]

Can be swapped out (Y,N,blank).: [ ]  Protection level (0-3)...........: [0]

Lotus 1-2-3 Release 3.1

-----------------------

Some users find that they get system hangs when switching back to Lotus 1-2-3

Release 3.1 from a DESQview menu or from another DESQview window.  There are two

already existing solutions to this problem: 1) Setting Keyboard Conflict to 8 in

the 1-2-3 window, which sometimes causes shift states to get stuck in the 1-2-3

window; and 2) the L4-LOAD.COM program that ships with DESQview 2.33 and later.

HOOKINT can be used as an equivalent to L4-LOAD.COM.  To do so, load 1-2-3 in a

DESQview window with a batch file that contains the following lines:

HOOKINT 9 82 /D

123 -C

If you are running on a monochrome system, substitute the -M parameter for the

-C on the 123 line in the batch file.  For further information on how to run

Lotus 1-2-3 Release 3.1 in DESQview, see the Quarterdeck technote LOTUS3.TEC.

Technical Background for HOOKINT

--------------------------------

The file HOOKINT.COM is a utility that uses DESQview's External Device Interface

(XDI) to guarantee that a particular software interrupt handler that is in

effect within a DESQview window will always be in place when that software

interrupt is called from inside a particular hardware interrupt handler.  This

can be important because hardware interrupts are often given to one program in

DESQview when the state of the computer is set up for a completely different

program.  If a program's hardware interrupt handler assumes that it can safely

call a given software interrupt, the program may not run successfully in

DESQview.  HOOKINT is designed to help such programs.

The syntax for HOOKINT is:

HOOKINT intnum [restorenum] [/D]

where:

intnum = interrupt number to hook

restorenum = interrupt number to make valid when

an INT intnum occurs

/D = Defer hooking until someone else does

For instance, the command HOOKINT C 66, issued inside a DESQview window, will

guarantee that the INT C (IRQ4, which is COM1) handler that is already in place

will be able to issue an INT 66 and call the INT 66 handler that is in effect in

that window.  Without this command, the INT C handler could unintentionally call

another window's INT 66 handler when it issues its INT 66, depending on which

window is receiving a time slice when the INT C occurs.

If you add a /D to this command (HOOKINT C 66 /D), then the INT 66 handler in

that window will be valid, not just when the INT C handler already in place is

called, but when the INT C handler that is about to be installed in that window

is called.  To know whether or not to use the /D parameter to HOOKINT, you need

to know which program is installing the hardware interrupt handler in question.

If this program is loaded before HOOKINT (or even before DESQview), the /D

parameter should not be used.  If this program is loaded after HOOKINT, the /D

parameter should be used.

If loaded before DESQview, HOOKINT simply grabs an interrupt, without performing

any of the functions that make it useful to applications.  Unless you wish to

use HOOKINT as a debugging tool, it should always be loaded in a DESQview

window.

END OF FILE