To run the IBM Screen-Reader with your Accent-SA, please follow the

instructions as described below:

1. Turn Accent-SA's volume to 1/3 clockwise.  Connect your PS2 COM1 port to

Accent-SA by using a straight-through RS-232 cable.  Make sure the DIP

switch on Accent-SA's back pannel has only position 4 and 8 ON; switched

to the top.  After you plug the AC adaptor into the wall and Accent-SA,

flip Accent-SA's On/Off switch to the ON position, Accent-SA should say

"ACCENT READY.".

2. Do not run the SETUP procedures as described in the IBM Screen-Reader

User's Guide.  Make a backup for your Screen-Reader diskette.  Prepare

to INSTALL the Screen-Reader in a designated Directory (hard disk) or

in a diskette for using Accent-SA as its synthesizer.

3. From the diskette provided, Copy ACCENT.TTS to the Installing

Directory or the Installing diskette as SRD.TTS.  Note that this will

overwrite the original SRD.TTS file.

4. From the diskette provided, Copy ACCENT.MSG to the Installing

Directory or the Installing diskette as SRD.MSG.  Note that this will

overwrite the original SRD.MSG file.

5. Run the INSTALL procedures as described in the IBM Screen-Reader User's

Guide.

6. Whenever you wish to run the Screen-Reader with Accent-SA, simply go

to the Installed Directory or diskette and type in SRD with a Carriage-

Return.  This may also be put into the AUTOEXEC.BAT file.

After you become familiar with Accent's operations and software commands,

you may tailor the ACCENT.TTS and ACCENT.MSG to meet your specific needs;

for example, since the audio volume is recommended to be controlled by

the volume control knob (leave the software volume at its maximum for the

best resolution), you may use the software volume steps for the speech rate

(Accent has 18 speech rates) or for the voice characteristics (or tone).

The file ACCENT1.TTS is an example of an alternative ACCENT.TTS.  If you

use the ACCENT1.TTS for the ACCENT.TTS, the software volume steps will be

used to change Accent's voice characteristics, and the pitch steps will be

used to change Accent's pitch only (note that in ACCENT.TTS the pitch steps

has pitch and voice characteristics parameters mixed).

###################################