FAX/DATA MODEM

USER'S MANUAL

Notice:

Bitcom is a registered trademark of BIT Software Inc.

BitFAX is a registered trademark of BIT Software Inc.

Crosstalk is a registered trademark of Microstuf Corporation.

Hayes is a trademark of Microcomputer Products Inc.

PCTALK is a registered trademark of Headlands Press Inc.

Smartcom is a trademark of Hayes Microcomputer Products.

Macintosh is a trademark of Apple Computer Corp.

Table of Contents

1-INTRODUCTION	4

2-COMMAND REFERENCE .	5

2.1 General Command Information 	6

2.2 AT Commands 	7

2.2.a Commands Preceded by & 	12

2.2.b Commands Preceded by \ 	17

2.2.c Commands Preceded by % 	19

2.2.d Commands Preceded by + 	21

2.2.e Commands Preceded by **	24

2.3 Dial Modifiers 	25

2.4 Commands Not Preceded By AT 	27

2.5 Result Codes	28

3-S REGISTER REFERENCE 	31

3.1 Register Summary 	32

3.2 Glossary of the S Registers 	35

4-TROUBLESHOOTING GUIDE 	52

5-TECHNICAL SPECIFICATIONS 	53

6-QUICK REFERENCE	56

7-GLOSSARY	58

8-ASCII CODE TABLE	61

1-INTRODUCTION

Congratulations on your purchase of this outstanding Fax/Data Modem. This manual describes how to operate your new Fax/Data Modem.

Instructions for installing your Fax/Data Modem will be found in the Installation Manual, while the information in this manual, deals exclusively with the operation of the modem after it is installed, such as the command set, the internal configuration registers, troubleshooting and testing.

Features:

These Fax/Data Modems combine the features of a 33600/28800/14400/9600 bps data modem and a 14400/9600 bps FAX modem. Your new Fax/Data Modem gives your personal computer the ability to send and receive FAX messages over the telephone line like a standard FAX machine. Your Fax/Data Modem also allows your PC to communicate with other personal computers, terminals or BBS's (Bulletin Board Systems) through the data modem functions.

When used as a data modem your Fax/Data Modem uses the standard AT command set and is fully compatible with ITU-T V.42, V.42 bis, V.34 (ANNEX 12), V.32 bis, V.32, V.22 bis, V.23, V.22, V.21, MNP2-5, Bell 103, 212A, and Rockwell V.FC. When used as a Fax/Data Modem it communicates with all ITU-T Group 3 FAX machines and is compatible with ITU-T V.27 ter and V.29, V.17, T.4 and T.30. Switching between DATA mode operation and FAX mode operation of your Fax/Data Modem is done through its firmware, no hardware settings are required.

If you are already familiar with the use of a modem and the Hayes AT command set, this modem will be extremely easy for you to use. Just read the Installation procedures in the installation manual and you are ready to begin operation. If you are new to modem communications, we recommend that you read through this manual first. If you come across terms that you don't understand, consult the glossary. Words in boldface type are command names, commands, or default settings. Carriage returns (Enter) are noted with <CR> or [ENTER]; this does not mean to enter these characters literally; but instead to press the Enter key.

The communication software which should be used depends on the kind of machine that you are going to communicate with. If you are going to call a FAX machine then you must use the Fax software. If the machine that you are going to communicate with is a modem then you must use a data modem communications software.

This manual is written to be used for several models of Fax/Data Modems. Some of the information in this manual may not apply to your fax/data modem.

2-COMMAND REFERENCE

This chapter provides an alphabetized reference with examples for all commands for the modem. The system of commands is depicted below in Figure 2-1.

To use these commands for dialing or configuring the modem, make sure the communications software package you will be using lets you operate the modem through its internal commands. If your software permits use of the modem's internal commands, read this chapter. If not, read your software user's manual and ignore the rest of this manual.

Commands

Commands not 	Commands not

Preceded by AT	Preceded by AT

A/ 	Escape	Standard AT	Extended AT

Command	Command 	Commands	Commands

Fig.2-1 System of Commands

2.1 General Command Information

Except for the A/ command and the +++ escape command described in Section 2.4, all commands must be prefixed with the attention code AT. For instance, the A command (below) would be entered as: "AT A <CR>". Without the AT prefix, the command line cannot be executed. Once entered, AT cannot be deleted with the Backspace or Delete key.

More than one command can be placed on a single line and, if desired, separated with spaces for readability. Once the carriage return (Enter) key is pressed, the command line is executed. A line with no carriage return is ignored.

The modem accepts either upper or lower case characters in the command line and ignores any spaces within or between commands. Typing errors can be corrected with the Backspace key. Exceptions are noted in the description of specific commands.

Variables (r and x) are listed in italics. Punctuation symbols (, ; ! @) use as dial modifiers are listed alphabetically according to their English names at the beginning of Section 2.3. Where two commands are separated by a slash, either command will have the same effect. For example, if the command is listed as B0/B, issuing either B0 or B will have the same effect.

2.2 AT Commands

A	Go On-line in Answer Mode

This command instructs the modem to go off-hook immediately and then make a handshake with the remote modem. Handshaking is not available during leased line operation.

A is usually used to manually answer an incoming call or to switch from voice conversation to data communication.

Bn	  Select Protocol to 300 bps or 1200 bps

B0 or B	  Selects ITU-T 300 or ITU-T 1200 protocol once the command line prefix AT has been      entered at the 300 bps or 1200 bps data rate

B1	  Selects BELL 300 or BELL 212A protocol once the command line prefix AT has been entered      at the 300 or 1200 bps data rate. (default)

Cn	Carrier Transmit Control

Controls the transmit carrier. The modem is preset to turn carrier on and off as necessary (the C1 option). The signal is on when the modem is calling, or connected to a remote modem, and is off when it is not. The C0 option is NOT valid.

C0	Not permitted; returns ERROR result code

C1	Normal transmit carrier switching (preset).

D	Go On-line in Originate Mode

D instructs the modem to go off-hook immediately and automatically dial the number contained in the dial string following D. The dial string may contain any of the dial modifiers contained in the following section. The D command without a dial string is usually used to switch from voice conversation to data communication or to call a remote modem in leased-line operation mode.

En	Command Echo

E0/E	Disables command echo

E1	Enables command echo (default)

Hn	Hang Up

H0/H	Goes on-hook (hangs up)

H1	Goes off-hook (ready to dial)

In	Identification

I0/I	Report the product code.

I1	Report the hardware checksum.

I2	Report "ERROR".

I3	Report firmware revision.

I4	Report OEM defined identifier string.

I5	Report the country code parameter.

I6	Report modem data pump model and internal code revision.

I7	Report the DAA code.

Ln	Control Speaker Volume

L0/L	Low volume.

L1	Low volume. (Default)

L2	Medium volume.

L3	High volume.

Mn	Monitor Speaker On/Off

M0/M	Speaker is always off

M1	Speaker is off while receiving carrier (default)

M2	Speaker is always on

M3	Speaker disabled while dialing or receiving carrier

Nn	Automode Enable

N0	Automode detection is disabled.

N1	Automode detection is enabled. this command is equivalent to F0. (Default)

On	Return to On-Line Data Mode

O0	Enters on-line data mode without a retrain. Handling is determined by the Call Establishment task. Generally, if a connection exists, this command connects the DTE back to the remote modem after an escape (+++).

O1	Enters on-line data mode with a retrain before returning to on-line data mode.

P	Set Pulse Dial as Default

Causes the modem to assume that all subsequent dial commands are pulse dials. You may omit the "P" from the dial strings.

Q	Result Code Display

Determines whether the modem sends the result codes to the DTE.

Q0	Allows the modem to send result codes to the DTE.

Q1	Prohibits the modem from sending result codes to the DTE.

Sn	Reading and Writing to S Registers

Sn?	Reads S Registers:

Reads the contents of the S register specified by 'n'

Sn=x	Writing to Registers:

Writes the value of x to the specified S register. All the registers will return the OK response if x is a legal value. However some registers will not actually write the value; these are: S1, S13-S15, S20-S24, and S27. (n=0-95, x=0-255)

T	Set Tone Dial as Default

Causes the modem to assume that all subsequent dial commands are tone dial.

Vn	Select Word or Digit Result Codes

V0/V	Displays result codes in digital format

V1	Displays result codes in verbose format (default)

Wn	Negotiation Progress Reporting

An additional set of result codes can be enabled with the W command to report progress of the negotiation phase of error-correction mode. These codes report the carrier speed (300 bps - 2400 bps) and the error correction protocol. These messages can also be reported in either numeric or verbose form. For example, both 77 and PROTOCOL: LAP-M indicate that the error correction protocol is LAP-M.

If the error correction link is negotiated and a connection is made, the appropriate CONNECT XXXX message reports another connection speed.

W0	Error correction call progress not reported.

W1	Error correction call progress reported.

W2	Error correction call progress not reported. CONNECT XXXX message reports DCE speed.

Xn	Extended Result Codes

This command selects which subset of the messages will be used by the modem to inform the DTE of the results of commands.

Blind dialing is enabled or disabled by country parameters. If the user wishes to enforce dial tone detection, a "W" can be placed in the dial string (see D command). Note that the information below is based upon the default implementation of the X results table.

If the modem is in facsimile mode (+FCLASS=1 or 2), the only message sent to indicate a connection is CONNECT without a speed indication.

X0	Disables monitoring of busy tones unless forced otherwise by country requirements; send only OK, CONNECT, RING, NO CARRIER, ERROR, and NO ANSWER result codes. Blind dialing is enabled/disabled by country parameters. If busy tone detection is enforced and busy tone is detected, NO CARRIER will be reported. If dial tone detection is enforced or selected and dial tone is not detected. NO CARRIER will be reported instead of NO DIAL TONE. The value 000b is written to S22 bits 6, 5, and 4, respectively.

X1	Disables monitoring of busy tones unless forced otherwise by country requirements; send only OK, CONNECT, RING, NO CARRIER, ERROR, NO ANSWER, and CONNECT XXXX (XXXX=rate). Blind dialing is enabled/disabled by country parameters. If busy tone detection is enforced and busy tone is detected, NO CARRIER will be reported instead of BUSY. If dial tone detection is enforced or selected and dial tone is not detected, NO CARRIER will be reported instead of NO DIAL TONE The value 100b is written to S22 bits 6, 5, and 4, respectively.

X2	Disables monitoring of busy tones unless forced otherwise by country requirements; send only OK, CONNECT, RING, NO CARRIER, ERROR, NO DIAL TONE, NO ANSWER, and CONNECT XXXX. If busy tone detection is enforced and busy tone is detected, NO CARRIER will be reported instead of BUSY. If dial tone detection is enforced or selected and dial tone is not detected, NO DIAL TONE will be reported instead of NO CARRIER The value 101b is written to S22 bits 6, 5, and 4, respectively.

X3	Enables monitoring of busy tones; send only OK, CONNECT, RING, NO CARRIER, ERROR, NO ANSWER, and CONNECT XXXX. Blind dialing is enabled/disabled by country parameters. If dial tone detection is enforced and dial tone is not detected, NO CARRIER will be reported. The value 110b is written to S22 bits 6, 5, and 4, respectively.

X4	Enables monitoring of busy tones; send all messages. The value 111b is written to S22 bits 6, 5, and 4, respectively. (Default.)

Yn	Enables or Disables Long Space Disconnect

Y0 or Y	Disables Long Space Disconnect (default)

Y1	Enables Long Space Disconnect.

Zn	Reset

Zn, which must be placed at the end of the command line, resets the active configuration of the modem to the stored configuration saved in nonvolatile RAM, hangs up the modem, and clears the command buffer.

Z0 or Z	Resets the modem and loads stored configuration 0

Z1	Resets the modem and loads stored configuration 1

2.2.a Commands Preceded by &.

&Cn	Select DCD Options

&C0/&C	Maintains an ON status for the Data Carrier Detect (DCD) (default)

&C1	Uses the actual state of the carrier from the remote modem for DCD

&Dn	DTR Option

Determines actions taken by the modem in relation to the Data Terminal Ready (DTR) signal of the serial port. The effect of DTR loss depends upon the combination of the &D and &Q commands, as follows:

1.	Modem disconnects and sends OK result code.

2.	Modem goes into command mode if already in data mode and sends the OK result code.

3.	Modem disconnects, sends the OK result code, and disables auto answer while DTR is OFF.

4.	Modem performs a warm start.

&D0	&D1	&D2	&D3

&Q0	none	2	3	4

&Q1	1		2	3	4

&Q2	3		3	3	3

&Q3	3		3	3	3

&Q4	1		2	3	4

&Q5	none	2	3	4

&Q6	none	2	3	4

&F	Fetch Factory Configuration

&F0	Recall factory profile 0 (default)

&F1	Recall factory profile 1

&Gn	Set Guard Tone

&G0 - G1	Disable guard tone (default)

&G2	Select 1800Hz guard tone

&Kn DTE/Modem Flow Control

Determines how the modem controls the flow of data between the local DTE and the modem. When the modem terminal buffer is nearly full, the modem will either send an XOFF or drop CTS to stop the data flow. When the buffer is nearly empty, the modem will either send an XON or raise CTS to start the data flow.

&K0	Disable DTE/DCE flow control.

&K3	Enable RTS/CTS DTE/DCE flow control.(Default for data modem).

&K4	Enables XON/XOFF DTE/DCE flow control.

&K5	Enables transparent XON/XOFF DTE/DCE flow control.

&K6	Enable RTS/CTS and XON/XOFF DTE/DCE flow control.

&Mn	Communication Mode

(Same as &Q0 - &Q3)

&Pn	Select Pulse Dialing Make/Break Ratio

&P0 or &P	Sets a 39/61 make/break ratio @ 10 pps - used in USA. (default)

&P1	Sets a 33/67 make/break ratio @ 10 pps

&P2	Sets a 39/61 make/break ratio @ 20 pps

&P3	Sets a 33/67 make/break ratio @ 20 pps

&Qn	Sync/Async Mode

This command is extension of the &M command and is used to control the connection modes permitted. It is used in conjunction with S36 and S48.

&Q0	Select direct asynchronous operation. The value 000b is written to S27 bits 3, 1, and 0 respectively.

&Q1	Select synchronous connect mode with async off-line command mode. The value 001b is written to S27 bits 3, 1, and 0 respectively.

&Q2	Select synchronous connect mode with async off-line command mode and enables DTR dialing of directory 0. The value 010b is written to S27 bits 3, 1, and 0 respectively.

&Q3	Select synchronous connect mode with async off-line command mode and enables DTR to act as Talk/Data switch. The value 011b is written to S27 bits 3, 1, and 0 respectively.

&Q5	The modem will try to negotiate an error-corrected link. The modem can be configured using S36 to determine whether a failure will result in the modem returning on-hook or will result in fallback to an asynchronous connection. The value 101b is written to S27 bits 3, 1, and 0, respectively. (Default)

&Q6	Select asynchronous operation in normal mode (speed buffering). The value 110b is written to S27 bits 3, 1, 0, and respectively.

&Rn	RTS/CTS Option

Controls the CTS signal in the synchronous mode. (CTS is always on in the synchronous mode, unless &K3 has been selected.)

&R0	CTS tracks RTS. CTS is turned ON in response to an OFF-to-ON transition from the local DTE after a delay period specified in register S26. (Default)

&R1	The modem ignores RTS; CTS is held ON unless &K3 has been selected

&Sn	DSR Option

Determines whether DSR operates in accordance with the EIA-232-D specification or remains ON

&S0	DSR is always ON. (Default)

&S1	DSR will become active after answer tone has been detected and inactive after the carrier has been lost.

&Tn Testing and Diagnostics (See Chapter 4)

Testing commands must be initiated in command mode (at the end of the command line) with asynchronous operation in the Direct mode, selected (&Q0) at a speed of 1200 bps or faster. A telco connection must be established prior to loopback tests. If these conditions are not met, an ERROR result code is issued. If a local analog loopback is initiated while the modem is connected, the modem disconnects before performing the test. A test remains active for the period of time specified in register S18. If S18 is zero, the test aborts when the user issues the &T0 command.

&T0 or &T	Terminates any test currently in progress

&T1	Initiates a local analog loopback in accordance with ITU-T V.54, L3; verifies the path between the local DTE and the local modem

&T2	Returns ERROR

&T3	Initiates a remote digital loopback locally. Characters received from the remote modem are looped back to the remote modem by the local modem

&T4	Grants a remote digital loopback request from the remote modem. (Default)

&T5	Denies a remote digital loopback request from the remote modem

&T6	Initiates a remote digital loopback in accordance with ITU-T V.54, L2; characters are transmitted to the remote modem and then looped back to the local modem

&T7	Initiates a remote digital loopback with self-test in accordance with ITU-T V.54, L2

&T8	Initiates a local analog loopback with self-test in accordance with ITU-T V.54, L2

&V	View Configuration Profiles

&V	Displays the active configuration profile

&Wn	Store the Current Configuration to Nonvolatile RAM

&W0	Writes the current active configuration to profile 0 in nonvolatile RAM

&W1	Writes the current active configuration to profile 1 in nonvolatile RAM

&Xn	Clock Source Selection

Selects the source or the synchronous transmit clock for the modem during synchronous operation.

&X0	Select Internal Timing: the modem generates its own transmit clock signal (Default)

&X1	Select External Timing: the local provides the transmit clock signal

&X2	Select Slave-Receive Timing: the modem derives the clock from the incoming carrier

&Yn	Select the Default Profile

&Y0 or &Y	Uses profile 0 on power-up (default)

&Y1	Uses profile 1 on power-up

&Zn	Store Telephone Numbers (n=0 to 3)

&Zn Store one of four dial strings (including a telephone number) of up to 45 digits in nonvolatile RAM.

For example: to store the telephone number 002852117 to RAM location 1, issue the following command:

Command: AT&Z1=002852117<CR>

2.2.b Commands Preceded by \

\An	Select Maximum MNP Block Size

The modem will operate an MNP error corrected link using a maximum block size controlled by the parameter supplied. The parameter value, if valid, is written to S40 bits 6 and 7.

\A0	64 characters.

\A1	128 characters. (Default.)

\A2	192 characters.

\A3	256 characters.

Result Codes:

OK	n=0 to 3.

ERROR	Otherwise.

\Bn Transmit Break to Remote

In non-error correction mode, the modem will transmit a break signal to the remote modem with a length in multiples of 100ms according to parameter specified. If a number in excess of 9 is entered, 9 is used. The command works in conjunction with the \K command.

In error correction mode, the modem will signal a break through the active error correction protocol, giving no indication of the length.

\B1 - \B9	Break length in 100ms units. (Default=3.) (Non-error corrected mode only.)

Result Codes:

OK	If connected in data modem mode.

NO CARRIER	If not connected or connected in fax modem mode.

Note:	When the modem receives a break from the remote modem, break is passed to the DTE as follows: In non-error correction mode direct, the break length is passed; in non-error correction mode normal and in error correction mode, a 300ms break is passed.

\Gn Modem to Modem Flow Control (XON/XOFF)

Enables or disables modem flow control during a Normal Mode connection. Since Reliable Mode has its own method of flow control, the \Gn command is ignored when error correction is selected. However, DTE-to-modem flow control remains active during reliable link.

\G0	Disable flow control (Default)

\G1	Enable flow control

\Kn	Break Control

Determines the modem's response when a BREAK is received from the DTE or the remote modem, according to the following conditions:

When a BREAK is received from the DTE during Normal or MNP Mode:

\K0,2,4	Modem enters Command Mode (waiting for an AT command) without sending a BREAK to the remote modem

\K1	Modem clears the terminal and modem buffers

\K3	Modem does not clear the buffers

\K5	Modem sends a BREAK to the remote modem in sequence with any transmitted data (Default)

When a BREAK is received from the remote modem during Normal Mode:

\K0,1	Modem clears the terminal and modem buffers

\K2,4	Modem does not clear the buffers

\K4,5	Modem sends a BREAK in sequence with any transmitted data

When a BREAK is received from the DTE during Direct Mode:

\K0,2,4	Modem sends a BREAK to the remote modem

\K1,3,5	Modem sends a BREAK to the remote modem

\Nn	Operation Mode Control

Selects the operating mode to be used during connection

\N0	Selects Normal (speed buffering) Mode. (Same as &Q6)

\N1	Selects Direct (pass-through) Mode. (Same as &Q0)

\N2	Selects Reliable Link Mode

\N3	Selects Auto-reliable Mode. Modem attempts to connect with error correction

\N4	Select LAPM error-correction mode

\N5	Select MNP error-correction mode

2.2.c Commands Preceded by %

%C	Enable/Disable Data Compression

Enables or disable data compression negotiation. The modem can only perform data compression on an error corrected link. The parameter value, if valid, is written to S41 bits 0 and 1.

%C0	Disables data compression. Resets S46 bit 1.

%C1	Enables MNP 5 data compression negotiation. Resets S46 bit 1.

%C2	Enables V.42 bis data compression. Sets S46 bit 1.

%C3	Enables both V.42 bis and MNP 5 data compression. Sets S46 bit 1. (Default.)

Result Codes:

OK	n=0,1,2, or 3.

ERROR	otherwise.

%En	Enable/disable Line Quality Monitor and Auto-Retrain or Fallback/Fall Forward

Control whether or not the modem will automatically monitor the line quality and request a retrain (%E1) or fall back when quality is insufficient or fall forward when line quality is sufficient (%E2). Applies to dial-up line only. The parameter value valid, is written to S41 bits 2 and 6. If enable, the modem attempts to retrain for a maximum of 30 seconds.

%E0	Disable line quality monitor and auto-retrain. (Default)

%E1	Enable line quality monitor and auto-retrain.

%E2	Enable line quality monitor and fallback/fall forward.

%E3	Enable line quality monitor and auto-retrain, but hang-up. Immediately when EQM reaches hang-up threshold. (fast hang-up)

%L	Report Received Signal Level

Returns a value identifying the received signal level. The possible values are:

009	–9 dBm

010	–10 dBm

011	–11 dBm

043	–43 dBm

%Q	Line Signal Quality

Reports the line signal quality (DAA dependent). Returns the higher order byte of the EQM value. Based on the EQM value, retrain or fallback/fall forward may be initiated if enabled by %E1 or %E2.

2.2.d Commands Preceded by +

Select Modulation

Coded-format command selects the modulation, optionally enables or disables automode, and optionally specifies the highest connection rates using up to four optional sub-parameters.

Command format is:

+MS=<mod>	[,[<automode>][,[<min_rate>][,[<max_rate>]]]]

+MS=<mod>	[,[<automode>][,[<min_rate>][,[<max_rate>][,[min_rx_rate][,		[<max_rx_rate>]]]]]]

Value as reported by the +MS? command (see below):

10, 2, 300, 33600

Parameters are defined as:

1.	<mod>= A decimal number which specifies the preferred modulation (automode enabled) or modulation (automode enabled) to use in originating or answering a connection. The options are:

<mod>	Modulation selected	Possible Rates (bps) (See option <automode>,

<min_rate>, and <max_rate> sub-parameters)	Notes

0		V.21		300

1		V.22		1200

2		V.22 bis		2400 or 1200

3		V.23		300

9		V.32		9600 or 4800

10		V.32 bis		14400, 12000, 9600, 7200 or 4800

11		V.34 		33600, 31200, 28800, 26400, 24000,

21600, 19200, 16800, 14400, 12000,

9600, 7200, 4800 or 2400			Default

64		BELL103	300

69		BELL212	1200

74		V.FC		28800, 26400, 24000, 21600, 19200,

16800, 14400

Notes: 	For V.23, originating modes transmit at 75 bps and receive at 1200 bps; answering modes transmit at 1200 bps and receive at 75 bps. The rate is always specified as 300 bps.

Modem may also automatically switch to another modulation (automode), subject to the following constraints:

The modem may not be able to automatically switch from the current modulation (specified by <mod>) to some other modulation. For example there is no standard way to automode from Bell 103 to V.23. The DTE may disable automode operation (see <automode> below). The DTE may constrain the range of modulations available by specifying the lowest and highest rates (see <min_rate> and <max_rate> below). The DTE may selectively disable some modulations by reading, editing, and writing the +MS parameter (see below).

2.	<automode> is an optional numeric value which enables or disable automatic modulation negotiation (e.g.,per V.8 for V.34 or ITU-T V.32 bis Annex A for V.32 bis). The options are:

<automode>	Option Selected						Notes

0		Automode disabled

1		Automode enabled without V.8 (except for V.34)

2		Automode enabled with V.8 for any applicable modulation	Default

The default value is enable as defined for the associated modulation (e.g., V.8 for V.34 and ITU-T V.32 bis for V.32 bis); however, there are modulations for which there is no automatic negotiation defined.

For <automode> = 0 (automode disabled, i.e., fixed modulation):

a.	If <max_rate> is within the rates supported by the selected modulation, the selected rated is that specified by <max_rate>. For example, AT+MS=9, 0, 1200, 4800 specifies V.32 4800 fixed rate.

b.	If <max_rate> is greater than the highest speed supported by the modulation specified by <mod>, the starting rate is the highest rate supported by the selected modulation. For example, AT+MS=9, 0, 2400, 14400 specifies V.32 9600 or 4800 bps.

c.	To mimic N0S37=x option, specify the <max_rate> and <min_rate> both to be the (same) requested speed, and <mod> to be the (known) modulation for that speed. For example, to select V.32 bis 12000 bps fixed mode (N0S37=10), issue AT+MS=9, 0, 12000, 12000.

For <automode> = 1 (automode enabled without V.8 except for V.34):

This mode is identical to the N1S37=x automode with a slight variation.

a.	If <max_rate> is greater than the highest rate supported by the modulation specified by <mod>, the modem automodes down from the highest rate of the selected modulation. For example, AT+MS=9, 1, 1200, 24000 specifies automoding down from V.32 bis 14400 bps.

b.	To mimic N1S37=x operation, specify the modulation and the rate to start automoding down from using <mod> and <max_rate>, respectively. Set <min_rate> to 300 to allow automoding all the way to V.21 300 bps.

For <automode> = 2 (automode enabled with V.8):

The modem connects at the highest possible rate.

3.	<min_rate> and <min_rx_rate> are optional numeric values which specify the lowest rate at which the modem may establish a connection. The value is decimal coded, in units of bps, e.g., 2400 specifies the lowest rate to be 2400 bps. The default is 300 bps.

4.	<max_rate> and <max_rx_rate> are optional numeric values which specify the highest rate at which the modem may establish a connection. The value is decimal coded, in units of bps, e.g., 14400 specifies the highest rate to be 14400 bps. The default is 33600 bps.

Selected Options

The modem can send a string of information to the DTE consisting of selected options.

+MS?

The response is:

+MS=<mod>, <automode>, <min_rate>, <max_rate>

For example,

+MS=9,1, 1200, 14400

Supported Options

The modem can send a string of information to the DTE consisting of supported options.

+MS=?The response is:

+MS=(list of supported <mod> values), (list of supported <automode> values), (list of supported <min_rate> values) (list of supported <max_rate> values)

For example,

+MS=(0,1,2,3,9,10,13,64,69,74),(0-2)(300-33600),(300-33600)

2.2.e Commands Preceded by **

**	Download to Flash Memory

The flash memory downloader in the modem firmware allows flash memory (e.g., ATMEL AT29C010 or compatible device) connected to the modem controller to be upgraded with revised firmware. This process transfers (uploads) the upgraded program from the host computer to the flash memory device. Note that this command apply only to the modem with flash memory.

The following sequence describes the flash download process:

a.	Initiate the download process using the AT**n command, where:

AT**0=Download speed is the last sensed speed.

AT**1=Download speed is 38.4k bps.

AT**2=Download speed is 57.6k bps.

The "Download initiated" message appears upon issuing the AT**n command.

b.	Perform an ASII upload of the new hex file from the host computer to the modem using an industry standard communications software of an equivalent process. Ensure that ASII transmit pacing is disabled in the communications package. Note that a flash download capability must be contained in the new download code to be enable subsequent flash downloads.

c.	A "Device successfully programmed" message is displayed at the completion of a successful download.

d.	A "Wrong Device" message is displayed if an EPROM is installed in the modem instead of a flash memory.

e.	A "Wrong Hex file or flow control" message is displayed if an incompatible hex file format is used (non-Motorola S3 format) or if the DTE ignores flow control (the flash download uses both Xon/Xoff and RTS/CTS flow control). If the wrong format was used, reinitiate the download at step a and upload a correctly formatted file in step b. If the DTE ignores flow control, reconfigure the DTE to support flow control and reinitiate the download at step a.

f.	If the download fails to complete (e.g., due to flash memory not responding correctly), initiate a modem power on reset. Upon modem reset completion, the downloader will automatically be restarted using a default download speed of 9600 bps.

2.3 Dial Modifiers

This section describes all of the dial modifiers which are used in dial strings.

@	Answer

"@", placed after a phone number, this modifier tells the modem to wait for 5 seconds of silence before dialing the next number in the dial string. @ is usually used to access a secure computer system that provides a silent answer as permission for further entrance.

,	Pause

",", placed anywhere in the dial string, tells the modem to pause for the number of seconds specified by S-register S8 before processing the rest of the dial string.

!	Initiate a Hookflash

"!", placed anywhere in the dial string, tells the modem to initiate a hookflash, which means to hang up for 0.5 seconds and then go off-hook again before processing the rest of the dial string. This modifier allows access to PBX features like call transferring.

;	Return to Command State after Dialing

";", which must be placed at the end of the dial string, returns to the command state after dialing the number placed ahead of it. A long telephone number would overflow the 40 character command buffer if placed all in one command line, so it must be broken into two or more command lines. Each part includes part of the number, and all but the last command line end with the ";" followed by a carriage return.

^	Tone Control

Toggles calling tone enable/disable:applicable to current dial attempt only.

J	Perform MNP

Perform MNP 10 link negotiation at 1200 bps (for this call only).

K	Enable Power Level

Enable power level adjustment during MNP 10 link negotiation (for this call only).

L	Re-dial Last Number

The modem will re-dial the last valid telephone number. The L must be immediately after the D with all the following characters ignored.

P	Pulse Dialing

P, placed ahead of a number, tells the modem to dial a number using pulse dialing.

S	Dial a Stored Number

S is used to dial one of four numbers stored in nonvolatile memory. For example, instead of entering a dial string, you can use this command:

Command: ATDTS=1<CR>

T	Touchtone Dialing

T, placed ahead of a number, tells the modem to dial a number using touchtone dialing.

W	Wait for Dialtone

W, placed after a number, tells the modem to wait up to 30 seconds to detect a one-second continuous dialtone before dialing the next number. W is most often used in a PBX system to wait for the dialtone of an outside telephone line.

2.4 Commands Not Preceded By AT

Two commands, A/ and +++, are neither preceded by the attention code AT nor followed by a carriage return.

A/	Repeat Command

A/ repeats the execution of the last command line stored in the command buffer. If the last command line is invalid, the ERROR result code will appear on the screen. Note that A/ cannot be preceded by AT; if it is, ERROR will appear on the screen.

+++	Escape

+++ returns to the on-line command state (command state without breaking the established connection) from the on-line state.

To escape, stop transmitting data, wait at least one escape guard time (the default time is one second), and then enter three consecutive escape characters (the default character is +). After one more escape guard time (one second), the modem returns to the command state and sends the OK result code to the screen. Note that the escape command is the only command that can be recognized by the modem in the on-line state; it cannot be recognized in the command state.

2.5 Result Codes

The modem sends a response to the user via the screen after a command is issued. As shown in the figure below, there are two forms for each result code: Verbose code and digit code.

Verbose				Digit		Description

OK				0		Command executed without errors

CONNECT				1		Connect to another modem

RING				2		Detect an incoming ring

NO CARRIER			3		Carrier lost or never detected

ERROR				4		Invalid command or invalid character

CONNECT 1200		5		Connection established at 1200 bps

NO DIALTONE			6		Dial tone not detected within timeout

BUSY				7		Detected a busy tone after dialing

NO ANSWER			8		No ringback or quiet answer was detected by the 						modem

CONNECT 600			9		Connection established at 600 bps

CONNECT 2400		10		Connection established at 2400 bps

CONNECT 4800		11		Connection established at 4800 bps

CONNECT 9600		12		Connection established at 9600 bps

CONNECT 7200		13		Connection established at 7200 bps

CONNECT 12000		14		Connection established at 12000 bps

CONNECT 14400		15		Connection established at 14400 bps

CONNECT 19200		16		Connection established at 19200 bps

CONNECT 38400		17		Connection established at 38400 bps

CONNECT 57600		18		Connection established at 57600 bps

CONNECT 115200		19		Connection established at 115200 bps

CONNECT 1200RX/75TX	22		V.23 connection established:transmit at 75 bps, receive 						at 1200 bps

CONNECT 75RX/1200TX	23		V.23 connection transmit at 1200 bps, receive at 75 						bps

DELAYED				24		Call fails to connect and number dialed is considered 						'delayed' due to country blacklisting requirements

BLACKLISTED			32		Call fails to connect and the number dialed is 						considered 'blacklisted'

FAX				33		A fax modem connection is established in a facsimile 						mode

DATA				35		A data modem connection is established in a facsimile 						mode

CARRIER 300			40		Carrier detected

CARRIER 1200TX/75RX V.23	44		Carrier detected

CARRIER 75RX/1200TX V.23	45		Carrier detected

Verbose				Digit		Description

CARRIER 1200			46		Carrier detected

CARRIER 2400			47		Carrier detected

CARRIER 4800			48		When the 4800 bps data rate in V.32 bis or V.32 mode 						has been detected on the line

CARRIER 7200			49		When the 7200 bps data rate in V.32 bis mode has been 						detected on the line

CARRIER 9600			50		When the 9600 bps data rate in V.32 bis or V.32 mode 						has been detected on the line

CARRIER 12000		51		When the 12000 bps data rate in V.32 bis mode has 						been detected on the line

CARRIER 14400		52		When the 14400 bps data rate in V.32 bis mode has 						been detected on the line

CARRIER 16800		53		When the 16800 bps data rate in V.34 mode has been 						detected on the line

CARRIER 19200		54		When the 19200 bps data rate in V.34 mode has been 						detected on the line

CARRIER 21600		55		When the 21600 bps data rate in V.34 mode has been 						detected on the line

CARRIER 24000		56		When the 24000 bps data rate in V.34 mode has been 						detected on the line

CARRIER 26400		57		When the 26400 bps data rate in V.34 mode has been 						detected on the line

CARRIER 28800		58		When the 28800 bps data rate in V.34 mode has been 						detected on the line

CONNECT 16800		59		Connection established at 16800 bps

CONNECT 21600		61		Connection established at 21600 bps

CONNECT 24000		62		Connection established at 24000 bps

CONNECT 26400		63		Connection established at 26400 bps

CONNECT 28800		64		Connection established at 28800 bps

COMPRESSION CLASS 5	66		MNP class 5 compression in use

COMPRESSION V.42BIS	67		V.42 bis compression in use

COMPRESSION NONE		69		No compression is being used

PROTOCOL:NONE		76		Normal mode selected

PROTOCOL:LAPM		77		LAPM mode selected

CARRIER 31200		78		When the 31200 bps data rate in V.34 mode has been 						detected on the line

CARRIER 33600		79		When the 33600 bps data rate in V.34 mode has been 						detected on the line

PROTOCOL:ALT		80

PROTOCOL:ALT-CELLULAR	81

CONNECT 33600		84		Connection established at 33600 bps

Verbose				Digit		Description

CONNECT 31200		91		Connection established at 31200 bps

+FCERROR				+F4

3-S REGISTER REFERENCE

Your modem has status registers. These registers are memory locations inside your modem which control your modem's operation. You usually do not have to worry about setting any register because the default values work for most applications.

The S registers are summarized in Fig. 3-1, along with their default values. Registers denoted with an "*" may be stored in one of the two user profiles by entering the &Wn command. One of these profiles may be loaded at any time by using the Zn command.

The factory default values are stored in ROM and are loaded into the active configuration at power-up or by the Zn command. In addition, the designated default profile is subsequently loaded, and may change some of the factory default values. The designated default profile can be changed by entering the &Yn command, where 'n' is one of the two possible user profiles. The factory defaults can be loaded at any time by entering the &F command.

3.1 Register Summary

The following chart summarizes your modem's registers:

Reg.#		Range		Unit		Dec		Hex	Description

S0		0-255		rings		0		00h	Number of rings 									before autoanswer

S1		0-255		rings		0		00h	Ring count

S2		0-255		ASCII		43		2Bh	Escape character code

S3		0-127		ASCII		13		0Dh	Command terminator 									(<CR> character)

S4		0-127		ASCII		10		0Ah	Line feed character

S5		0-255		ASCII		8		08h	Backspace character

S6		2-255		seconds		2		02h	Wait time for blind 									dialing

S7		1-255		seconds		50*		32h	Wait time for carrier after dial

S8		0-255		seconds		2*		02h	Pause time for comma

(dial delay)

S9		1-255		1/10 sec.	6*		06h	Carrier detect response 									time

S10		1-255		1/10 sec.	14*		0Eh	Lost carrier to hang-up 									delay

S11		50-255		1/1000 sec.	95		5Fh	DTMF Tone Duration

S12		0-255		1/50 sec.	50*		32h	Escape code timing

S13				Not used

S14		[Bit Mapped 		Options]*			138		8Ah

S15				Not used

S16		[Modem Test

Mode Register]

S18		0-255		seconds		0*		00h	Test mode timer

S19						0			Autosync Option

S20		0-255				0			Autosync HDLC Address or 									BSC Sync Character

S21		[Bit Mapped 		Options]			4*		04h	V.24/General bit-mapped 									options

Reg.#		Range		Unit		Dec		Hex	Description

S22		[Bit Mapped 		Options]			117*		75h	Speaker/Results Bit-mapped 									options

S23		[Bit Mapped 		Options]			54		36h

S24		0-255		seconds		5		00h	Sleep Inactivity Timer

S25		0-255		seconds		5*		05h	Delay to DTR

S26		0-255		1/100 sec	1*		01h	RTS to CTS delay

S27		[Bit Mapped 		Options]			9*		09h

S28		[Bit Mapped 		Options]			0*		00h

S29		0-255		10 ms		0*		00h	Flash Dial Modifier 									Time

S30		0-255		10 s		0*		00h	Disconnect Inactivity 									Timer

S31		[Bit Mapped 		Options]			2*		02h

S32		0-255		ASCII		17		11h	XON Character

S33		0-255		ASCII		19		13h	XOFF Character

S36						7*			LAPM Failure Control

S37						0*			Desired DCE Speed

S38		0-255		seconds		20*			Delay before forced 									disconnect

S39						3		03h	Flow Control

S40						105		69h	Bit Mapped Options

S41						3		03h	Bit Mapped Options

S46						138*		8Ah	Data Compression 									Control

S48						7*		07h	V.42 negotiation action

S82						128*		80h	LAPM Break Handling Option

S86		0-255							Call Failure Reason Code

S91		0-15				dBm		10	PSTN Transmit Attenuation Level

Reg.#		Range		Unit		Dec		Hex	Description

S92		0-15				dBm		10	Fax Transmit Attenuation Level

S95		[Bit mapped]			0*		00h	Extended result codes

Fig. 3-1 S-Register Summary

3.2 Glossary of the S Registers

S0	Number of Rings Before Auto Answer

S0 determines the number of rings that must be received before the modem automatically answers an incoming call. For example, when S0=3, the modem automatically answers after the third ring. When S0=0, the modem does not automatically answer an incoming call; it stays on-hook until the A command is issued manually to answer the incoming call.

Range:	0-255 rings

Default:	0

S1	Ring Count

S1 automatically increments its value by one each time the modem receives a ring while in the command state. S1 is reset to zero if no ring is detected within 8 seconds.

Range:	0-255 rings

Default:	0

S2	ASCII Value of Escape Character

S2 stores the ASCII value of the escape character. Setting register S2 to a value greater than 127 disables the escape command and you cannot return to the command state. With escape disabled, in the on-line state the modem cannot hang up until the power is turned off or the remote modem hangs up.

Range:	0-255, ASCII decimal.

Default:	43 (+)

S3	ASCII Value of Carriage Return

S3 stores the ASCII value of the carriage return character. (Pertains to asynchronous operation only.)

Range:	0-127, ASCII decimal.

Default:	13

S4	ASCII Value of Line Feed Character

S4 stores the ASCII value of the line feed character, if your computer does not recognize the default as a line feed, change the value. A value greater than 127 disables the line feed. When disabled, the line feed character that precedes or follows a result code is canceled. (Pertains to asynchronous operation only.)

Range:	0-127, ASCII decimal.

Default:	10 (Line Feed)

S5	ASCII Value of Backspace Character

S5 stores the ASCII value of the backspace character. The backspace is used to edit a command line. If our computer does not recognize the default as a backspace, change the value. (Pertains to asynchronous operation only.)

Set S5 to any value from 0 to 31 or 127. Do not set it to any value from 32 through 126 because these values correspond to printable ASCII characters. A value greater than 127 disable the backspace and makes it impossible to edit a command line.

Range:	0-32, ASCII decimal.

Default:	8 (Backspace)

S6	Wait Time before Blind Dialing

S6 controls how long the modem waits after it goes off-hook before it dials the first digit of the telephone number. The modem always pauses for at least 2 seconds, even if S6 is set to less than two seconds.

Range:	2-255 seconds.

Default:	2

S7	Wait for Carrier after Dial

S7 controls how long the modem waits for a carrier signal from a remote modem after originating a call or from the calling modem after going off-hook when answering a call.

S7 also controls how long the modem waits for a one-second continuous dialtone after dialing a number followed by the W dial modifier. If the modem detects a one-second continuous dialtone within the specified wait time, it proceeds to dial.

Range:	1-255 seconds.

Default:	50

S8	Pause Time for Comma

S8 controls how long the modem pauses when a comma "," is encountered in a dial string while executing a dial command.

Range:	0-255 seconds.

Default:	2

S9	Carrier Detect Response Time

S9 determines how long a carrier signal must be present for the modem to confirm it. The longer the response time, the easier it is for the modem to correctly recognize a carrier without mistaking other signals or transient noise on the line as a carrier.

Range:	1-255 tenths of a second.

Default:	6 (0.6 second)

S10	Delay between Loss of Carrier and Hang-Up

S10 determines the delay time between the loss of a carrier from the remote modem and hang-up. This allows for a temporary loss of carrier without causing the local modem to disconnect. When S10 is set to 255, the modem functions as if a carrier is always present.

The actual interval the modem waits before disconnecting is the value in S10 minus the value in S9. Therefore, the value in S10 must be greater than that in S9, or else the modem disconnects before it recognizes the carrier.

Range:	1-255 tenths of a seconds.

Default:	14 (1.4 seconds)

S11	DTMF (Touch-tone) Tone Duration

S11 determines the duration and spacing of tones for Dual Tone Multifrequency (DTMF) dialing. This value has no effect on pulse dialing.

Range:	50-255 millisecond.

Default:	95

S12	Escape Guard Time

S12 determines the escape guard time. The escape guard time is the minimum waiting time required before and after entering the escape code (three consecutive escape characters) in the on-line state. It is also the maximum waiting time allowed between any two consecutive escape characters. If the waiting time before or after the escape code is shorter than the guard time, or if the waiting time between consecutive escape characters is longer than the guard time, then the modem does not recognize the escape command and stays on-line.

If the escape guard time is set at 0 second, it is impossible to return the modem to command state.

Range:	0-255 1/50 of a seconds.

Default:	50 (1 second)

S13	Reserved

S14	Bit Mapped Options

Default:	138 (8Ah) (10001010b)

Bit			Value		Description

bit 1 (E)		0		Disabled

1		Enabled (default)

bit 2 (Q)		0		Disabled (default)

1		Enabled

bit 3 (V)		0		Numeric

1		Verbose (default)

bit 4				Reserved

bit 5 (P or T)	0		T (Tone dial) (default)

1		P (Pulse dial)

bit 6				Reserved

bit 7		0		Answer mode

1		Originate mode

S15	Reserved

S16	Modem Test Mode Register

Bit		Value		Description

bit 0	0		Local analog loopback terminated (default)

1		Local analog loopback in progress (&T1)

bit 1			Unused

bit 2	0		Local digital loopback terminated (default)

1		Local digital loopback in progress (&T3)

bit 3	0		Loopback activated by the remote modem terminated 					(default)

1		Loopback activated by the remote modem in progress (a remote 					digital loopback request from the remote modem)

bit 4	0		Remote digital loopback terminated (default)

1		Remote digital loopback in progress (&T6)

bit 5	0		Remote digital loopback-with self-test terminated 					(default)

1		Remote digital loopback-with self-test in progress (&T7)

bit 6	0		Local analog loopback with self-test terminated (default)

1		Local analog loopback with self-test in progress (&T8)

bit 7			Unused

S17	Reserved

S18	Test Timer

The test timer determines how long tests are performed. The modem automatically ends the test when the test time is expired. The default value of 000 disables the test timer. When the timer is thus disabled, all tests must be ended with the &T0 command.

Range:	0-255 seconds.

Default:	0

S19	AutoSync Bit Mapped Options

Defines the options for AutoSync operation (see &Q4 command). S19 must be set to the desired value before &Q4 is issued.

Default:	0

bit 0	Reserved

bit 1	BSC/HDLC format select

0 = BCS selected (Default)

1 = HDLC selected

bit 2	Address detection enable/disable

0 = disable (Default)

1 = Enable

bit 3	NRZI/NZI coding select

0 = NRZI (Default)

1 = NZI

bit 4	Idle indicator select

0 = Mark idle (Default)

1 = Flag or sync idle

bit 5-7	Reserved

S20	AutoSync HDLC Address or BSC Sync Character

Defines the HDLC address (S19 bit =1) or BSC Sync Character (S19 bit 1 =0) for AutoSync operation (see &Q4 command). S20 must be set to the desired value before &Q4 is issued

S21	V.24/General Bit Mapped Options

Default:	4 (00000100b)

Bit		Value		Description

bit 0	0		&J0 (default)

1		&J1

bit 2	0		&R0

1		&R1 (default)

bit 3,4	00		&D0 (default)

01		&D1

10		&D2

11		&D3

bit 5	0		&C0 (default)

1		&C1

bit 6	0		&S0 (default)

1		&S1

bit 7	0		Y0 (default)

1		Y1

S22	SPEAKER/Results Bit Mapped Options

Default:	117 (75h) (01110101b)

Bit		Value		Description

bit 0,1	00		Select L0

01		Select L1 (default)

10		Select L2

11		Select L3

bit 2,3	00		Select M0

01		Select M1 (default)

10		Select M2

11		Select M3

bit 4,5,6	000		Select X0

001		Reserved

010		Reserved

011		Reserved

100		Select X1

101		Select X2

110		Select X3

111		Select X4 (default)

bit 7			[RESERVED]

S23	Bit Mapped Options

Default:	55(37h) (00110111b)

Bit		Value		Description

bit 0	0		&T5

1		&T4 (default)

bit 1,2,3	000		Communications rate = 300 bps

001		Communications rate = 600 bps

010		Communications rate = 1200 bps

011		Communications rate = 2400 bps (default)

100		Communications rate = 4800 bps

101		Communications rate = 9600 bps

110		Communications rate = 19200 bps

111		Communications rate = 38400 bps or higher

bit 4,5	00		Even parity selected

01		Not used

10		Odd

11		None (default)

bit 6,7	00		&G0 (default)

01		&G1

10		1800Hz (&G2)

S24	Sleep Inactivity Timer

Set the length of time, in units of 10 seconds, that the modem will operate in normal mode with no detected telephone line of DTE line activity before entering low-power sleep mode.

Range = 0-255 (seconds)

Default = 5

S25	Delay to DTR

S25 serves two purposes. When the modem is operating in synchronous mode 1, the value assigned to S25 specifies the length of time the modem waits after a connection has been mode before examining DTR. This allows the modem to ignore an ON-to-OFF transition of DTR, giving the user time to disconnect the modem from the asynchronous terminal and attach it to a synchronous terminal, without forcing the modem back to the asynchronous command mode. During this time, the value for S25 is read in whole seconds.

In all other modes, and after call establishment in synchronous modes I and 4, the value is read in 1/100 seconds. In any mode, a change in DTR ( ON or OFF) that persists for a period shorter than the value held in S25 is ignored by the modem while it is in data mode.

Range:	0-255 (1 second for synchronous mode 1;0.01 second otherwise)

Default:	5

S26	RTS to CTS Delay Interval

Pertains to synchronous operation only. When CTS tracks RTS (&R0) and the modem detects an ON-to-OFF transition on RTS, this register sets the time delay before the modem turns CTS to ON.

Range:	0-255 hundredths of a second

Default:	1

S27	Bit Mapped Options

Default:	9(09h) (00001001b)

Bit		Value		Description

bit 0,1,3			Synchronous/asynchronous

310		selection (&Mn/&Qn)

000		&M0 or &Q0

001		&M1 or &Q1

010		&M2 or &Q2

011		&M3 or &Q3

100		&Q4

101		&Q5 (default)

110		&Q6

bit 2			Leased line control (&Ln)

0		Dial up line (&L0) (default)

1		Leased line (&L1)

bit 4,5			Internal clock select (&Xn)

00		Internal clock (&X0) (default)

01		External clock (&X1)

10		Slave clock (&X2)

bit 6			ITU-T/Bell mode select (Bn)

0		ITU-T mode (B0) (default)

1		Bell mode (B1)

S28	Bit Mapped Option Status

Default:	0

bit 0,1	Reserved

bit 2	Reserved

bit 3,4	Pulse dialing (&Pn)

0=39%-61% make/break ratio at 10 pulse per second (&P0) (default)

1=33%-67% make/break ratio at 10 pulse per second (&P1)

2=39%-61% make/break ratio at 20 pulse per second (&P2)

3=33%-67% make/break ratio at 20 pulse per second (&P3)

bit 5	Reserved

bit 6-7	MNP Link Negotiation Speed (*Hn)

0 = Link negotiation at highest speed (*H0) (default)

1 = Link negotiation at highest speed (*H1)

2 = Link negotiation at highest speed (*H2)

S29	Flash Dial Modifier Time

Range:	0-255 tenths of a second

Default:	0 (disable)

S30	Inactivity Disconnect Timer

This register determines the length of time, in seconds, the modem waits before disconnecting when no data is sent or received, This only operates in &Q5 and &Q6 modes, a value of 0 in this register disables the timer.

Range:	0-255 tenths of a second

Default:	0 (disable)

S31	Bit Mapped Options

Default:	2 (00000010b)

Bit/Value	Description

bit	1	Controls auto line speed detection (Nn)

0	Disable (N0)

1	Enable (N1) (default)

bit 	2,3	Controls error correction progress messages (Wn)

00	DTE speed only (W0) (default)

01	Full reporting (W1)

10	DCE speed only (W2)

S32	XON Character

Range:	0-255, ASCII decimal

Default:	17

S33	XOFF Character

Range:	0-255, ASCII decimal

Default:	19

S36	LAPM Failure Control

This register is read when the S48 register contains the value 128 or if an attempted error-correction link fails. These fallback options are initiated immediately upon connection if S48 = 128

Bit		Value		Action

bit 0,1,2	000		Modem disconnect.

001		Modem stays on-line and Direct mode connections 					established.

010		Reserved

011		Modem stays on-line and a Normal mode connection is 					established.

100		An MNP connection is attempted and if it fails, the modem 					disconnects.

101		An MNP connection is attempted and if it fails, a direct mode 					connection is established.

110		Reserved

111		An MNP connection is attempted and if it fails, the normal 					mode connection established. (default)

S37	Desired Line Connection Speed

Default:	0

bit 0-4	Desired line connection speed. This is interlinked with the Fn 			command. If an invalid number is entered, the number is accepted into the 			register, but S37 will act as if the default value has been entered.

0 = Attempt auto mode connection. (default)

1-3 = Attempt to connect at 300 bps.

4 = Reserved

5 = Attempt to connect at V.22 1200 bps.

6 = Attempt to connect at V.22 bis 2400 bps.

7 = Attempt to connect at V.23.

8 = Attempt to connect at V.32 bis/V.32 4800 bps.

9 = Attempt to connect at V.32 bis/V.32 9600 bps.

10 = Attempt to connect at V.32 bis 12000 bps.

11 = Attempt to connect at V.32 bis 14400 bps.

12 = Attempt to connect at V.32 bis 7200 bps.

bit 5-7	Reserved

S38	Delay Before Forced-Disconnect

This register controls how long the modem waits, after receiving the ATH command or loss of DTR, before it disconnects from the telephone line. When connected in an error-corrected mode, you can use this register to ensure that all data is transmitted from the modem's buffer before the modem disconnects.

If you set S38 between 0 and 254, the modem waits that number of seconds for the remote modem to acknowledge all data before it disconnects, If you set S38 to 255, the modem waits indefinitely for the remote modem to acknowledge all data.

Range:	0-255 seconds

Default:	0

S39	Flow Control

Default:	3(00000011b)

Bit		Value		Connect speed

bit 0-2	000		No flow control.

011		RTS/CTS &K3 (default)

100		XON/XOFF &K4

101		Transparent XON &K5

110		Both Method &K6

bit 3-7			Reserved

S40	General Bit Mapped Options

Default:	105 (69h) (01101001b)

Bit		Value		Description

bit 0-1			MNP Extended Services (-Kn)

0		Disable extended services (-K0)

(Default for non MNP 10 models)

1		Enable extended services (-K1)

(Default for MNP 10 models)

2		Enable extended services (-K2)

bit 2			Power level Adjustment for Cellular Use ( )Mn)

0		Auto-adjustment ( )Mo) (default)

1		Force adjustment ( )M1)

bit 3-5			Break handling (\Kn)

0		/K0

1		/K1

2		/K2

3		/K3

4		/K4

5		/K5 (default)

bit 6-7			MNP block size (\An)

0		64 chars (\A0)

1		128 chars (\A1) (default)

2		192 chars (\A2)

3		256 chars (\A3)

S41	Bit-Mapped Options

Default:	131 (10000011b)

Bit		Value		Description

bit 0,1			Compress selection %Cn

00		Disable %C0

01		MNP 5 %C1

10		V.42 bis %C2

11		MNP 5 and V.42 bis (%C3) (default)

bit 6,2			Auto retrain %En

00		Retrain disable %E0 (default)

01		Retrain enable %E1

10		Fallback/fall forward enable (%E2)

bit 3			Modem-to-modem flow control \G

0		Disable \G (default)

1		Enable \G1

bit 4			Block mode control \Ln

0		Store mode \L0 (default)

1		Block mode \L1

bit 5			Reserved

bit 7			Enable fallback to V.22bis/V.22(-Qn)

0		Disable (-Q0)

1		Enable (-Q1) (default)

S46	Protocol Selection

This register controls whether the modem tries to use data compression when the modem establishes an error corrected link. The default is for the modem to use data compression (138).

Default:	138

Value	Description

136		Execute error correction protocol with no compression.

138		Execute error correction protocol with compression. (default)

S48	V.42 Negotiation Action

Value	Description

0		Disable negotiation: bypass the detection and negotiation phases

7		Enable negotiation. (default)

128		Disable negotiation: bypass the detection and negotiation phases

S82	Break Handling Option

S82 is for compatibility purposes only, changing this register will not any affect.

S86	Call Failure Reason Code

When the modem issues a NO CARRIER result code, a value is written to this S-Register to help determine the reason for the failed connection. S86 records the first event that contributes to a NO CARRIER message. The cause codes are:

Range:	0,4,5,9,12,13, or 14

Default:

S86= 0	Normal disconnect, no error occurred.

S86= 4	Loss of carrier.

S86= 5	V.42 negotiation failed to detect an error-correction modem at the other end.

S86= 9	The modems could not find a common protocol.

S86= 12	Normal disconnect initiated by the remote modem.

S86= 13	Remote modem does not respond after 10 retransmission of the same message.

S86= 14	Protocol violation.

S91	PSTN Transmit Attenuation Level

Sets the transmit attenuation level from 0 to 15 dBm for the PSTN mode resulting in a transmit level from 0 to –15 dBm.

Range:	0 to 15 dBm (Corresponding to 0 to –15 dBm transmit level).

Default:	10 (–10 dBm transmit level).

S92	Fax Transmit Attenuation Level

Sets the transmit attenuation level from 0 to 15 dBm for the fax mode resulting in a transmit level from 0 to –15 dBm.

Range:	0 to 15 dBm (Corresponding to 0 to –15 dBm transmit level).

Default:	10 (–10 dBm transmit level).

S95	Extended Result Codes

This register can override some of the W command options. Set the appropriate bit to 1 to enable the corresponding result code, regardless of the W command setting Set the bit to 0 to disable the result code.

Bit		Description

0		CONNECT indicates DCE speed.

1		Append /ARQ to the CONNECT result code if the protocol is other than 			NONE.

2		CARRIER result code.

3		PROTOCOL: result code.

4		Reserved

5		COMPRESSION: result code

6		Reserved

7		Reserved

4-TROUBLESHOOTING GUIDE

This chapter describes steps you can take to resolve problems if they occur with the settings, functions, or normal operation of the modem.

No Response From Your Modem

Is the modem installed?

If the modem is not installed, refer to the on-line help system and follow the instructions for installing the modem.

Is the modem setup so it can be used?

This system works by switching the COM setting between COM port and Modem port. Use the ROM setup utility or the MobilePRO utility to check that the modem is setup correctly.

Check the settings in your communications software.

You modem does not connect after dialing, or does not function normally after connecting

Make sure that the telephone cable and telephone line are working normally.

Make sure that the telephone number you are calling is correct.

Make sure that the communication settings are correct.

If you are using a non-standard telephone line, try the number again after issuing the ATX3<CR> command to tell the modem to disable dialtone detection when dialing the number.

There may be a problem with the modem settings. Issue the ATZ<CR> or AT&F&W<CR> command to reset your modem and try again.

5-TECHNICAL SPECIFICATIONS

5.1 Features

Date Rates (bps)

300, 1200, 2400, 4800, 7200, 9600, 12000, 14400, 19200, 21600, 24000, 26400, 28800, 31200, 33600

Compatibility

(Data)	ITU-T V.34, V.32 bis, V.32, V.22 bis , V.23, V.22 and V.21, Bell: 212A and 103, Rockwell V.Fast Class

(Fax)	ITU-T V.17, V.29, V.27 ter, and V.21 ch2

Command Set	Enhanced 'AT' command set, Fax Class 1 & Class 2 command

Flow Control	Xon/Xoff, Hardware RTS/CTS

Data Compression	V.42 bis, MNP 5

Error Correction	V.42, MNP 2-4

Modulation

Data	33600 bps		TCM

31200 bps		TCM

28800 bps		TCM

26400 bps		TCM

2400 bps		TCM

21600 bps		TCM

19200 bps		TCM

16800 bps		TCM

14400 bps		TCM

9600 bps		TCM/QAM

7200 bps		TCM

4800 bps		QAM

2400 bps		DPSK

1200 bps		DPSK

1200TX/75RX bps	FSK

75 TX/1200RX bps	FSK

600 bps	DPSK

300 bps	FSK

FAX	14400 bps		QAM

9600 bps		QAM

7200 bps		QAM

4800 bps		QAM/DPSK

2400 bps		DPSK

Operation	Full-or Half-duplex 28800 bps with 26400, 24000, 21600, 19200, 14400, 9600, 4800, 2400 and 1200 bps auto fallback

Test Modes	Analog loopback, local digital loopback, and remote digital loopback

Audio Monitor	Built-in speaker, with software-controllable volume control

Guard Tone	550/1800 Hz

Compatibility:

Data	V.Fast Class		28800/26400/24000/21600/19200/16800/ 14400

V.34			33600/31200/28800/26400/24000/21600/ 						19200/16800/14400

ITU-T V.32 bis		14400/9600 bps. asynchronous, synchronous

ITU-T V.32		9600/4800 bps, asynchronous, synchronous

ITU-T V.22 bis		2400/1200 bps, asynchronous, synchronous

ITU-T V.23		1200/75 bps, asynchronous, synchronous

ITU-T V.22		1200 bps, asynchronous, synchronous

ITU-T V.21		300 bps, asynchronous

Bell 212A		1200 bps, asynchronous, synchronous

Bell 103			300 bps, asynchronous

Fax		ITU-T V.17		14400/9600 bps, half-duplex

ITU-T V.29		9600/7200 bps, half-duplex

ITU-T V.27 ter		4800/2400 bps, half-duplex

ITU-T V.26 bis		2400/1200, half-duplex

ITU-T V.26 A		2400, half-duplex

G-III	(Group 3 fax)

Compliance	FCC 15&68

Command Buffer	40 characters

Extra Memory	4 x 36 digits

Transmit Level	–11(+/–1) dBm

Receive Sensitivity	–40 dBm

Pulse Dialing Specifications

USA		International

Make/Break Ratio		39/61		33/67

Break Length		61 ms		67 ms

Dial Pulse Length		100 m		100 ms

Dial Pulse Rate		10 pps		10 pps

Interdigit Time		800 ms		800 ms

6-QUICK REFERENCE

A	Go On-line in Answer Mode

Bn	Select Protocol to 300 bps or 1200 bps

Cn	Carrier Transmit Control

D	Go On-line in Originate Mode

En	Command Echo

Hn	Hang Up

In	Identification

Ln	Control Speaker Volume

Mn	Monitor Speaker On/Off

Nn	Automode Enable

On	Return to On-line Data Mode

P	Set Pulse Dial as Default

Q	Result Code Display

Sn	Reading and Writing to S Registers

T	Set Tone Dial as Default

Vn	Select Word or Digit Result Codes

Wn	Negotiation Progress Reporting

Xn	Extended Result Codes

Yn	Enables or Disables Long Space Disconnect

Zn	Reset

&Cn	Select DCD Options

&Dn	DTR Option

&F	Fetch Factory Configuration

&Gn	Set Guard Tone

&Kn	DTE/Modem Flow Control

&Mn	Communication Mode

&Pn	Select Pulse Dialing Make/Break Ratio

&Qn	Sync/Async Mode

&Rn	RTS/CTS Option

&Sn	DSR Option

&Tn	Testing and Diagnostics (See Chapter 4)

&V	View Configuration Profiles

&Wn	Store the Current Configuration to Nonvolatile RAM

& Xn	Clock Source Selection

&Yn	Select the Default Profile

&Zn	Store Telephone Numbers (n=0 to 3)

\An	Select Maximum MNP Block Size

\Bn	Transmit Break to Remote

\Gn	Modem to Modem Flow Control (XON/XOFF)

\Kn	Break Control

\Nn	Operating Mode Control

%Cn	Enable/Disable Data Compression

%En	Enable/Disable Line Quality Monitor and Auto-Retrain or Fallback/Fall Forward

%L	Report Received Signal Level

%Q	Line Signal Quality

+MS	Select Modulation

**	Download to Flash Memory (For models with flash Memory only)

@	Answer

'	Pause

!	Initiate a Hookflash

;	Return to Command State after Dialing

^	Tone Control

J	Perform MNP

K	Enable Power Level

L	Re-dial Last Number

P	Pulse Dialing

S	Dial a Stored Number

T	Touchtone Dialing

W	Wait for Dialtone

A/	Repeat Command

+++	Escape

7-GLOSSARY

ASCII - An acronym for American Standard Code for Information Exchange. ASCII is a seven-bit code which defines 128 standard characters, including control characters, letters, numbers, and symbols. An extra 128 characters comprise the extended ASCII set.

Baud Rate - The transmission rate between two serial devices, e.g., modems, fax machines, etc. Measured in Bits Per Second.

Blind Dialing - In blind dialing, the modem continues to dial, regardless of the existence of a dialtone, ring, or busy signal.

BPS - Bits Per Second; the number of bits that can be transmitted in one second.

Carrier Signal - The analog data signal that a modem sends over telephone wires.

COMx - Where (x = 1, 2, 3, or 4), COMx is the name (address) of serial communications ports on personal computers. Each serial port in a personal computer has a different number.

CTS - Clear To Send

Default - The assumed value that is used for a command parameter when no other value is explicitly provided.

DCD - Data Carrier Detect.

DCE - Data Communication Equipment

DTE - Data Terminal Equipment

DTMF - Dual Tone Multifrequency (for touchtone dialing).

DTR - Data Terminal Ready.

FSK - Frequency Shift Keying

Make/Break Ratio - The ratio of the off-hook (make) to on-hook (break) interval is the make/break ratio in pulse dialing.

Modem - A combination of the words MOdulator and DEModulator. Modems transform digital data into analog signals and back again.

Nonvolatile Memory - An area of memory inside the modem where the default configuration profile is stored. Values recorded in this memory will not be lost when the power is turned off.

Off-Hook - The condition when the modem has picked up the telephone line.

Off-Line Command State - A modem state in which the modem accepts, interprets and executes commands from an asynchronous computer or terminal.

On-Hook - The condition when the modem has not picked up the telephone line; the telephone is hung up.

On-Line - A carrier signal link with a remote modem has been established; communication is in progress.

On-Line State - A modem state in which the modem is connected with a remote modem. Data can be sent or received from the remote modem in this state. No commands will be accepted from the modem except the escape command which will bring the modem into the on-line command state.

On-Line Command State - A modem state in which the modem can accept or execute commands from an asynchronous computer or terminal while remaining connected with the remote modem. The user can return the modem to the on-line state by issuing the AT0n command or put it into the off-line command state by issuing command such as ATZ or ATH.

Parity - An error-checking method by which the modem verifies that the data just sent is correct.

pps - Pulse per second

Profile - A list of default settings.

Protocol - A technical specification for serial communications; the protocols supported by the modem are listed in Appendix B.

PSK - Phase Shift Keying

Pulse Dialing - A dialing form in which each digit is represented by a series of pulses. Rotary Telephones all use pulse dialing.

QAM - Quadrature Amplitude Modulation

Result Codes - The response the modem returns to the screen upon executing a command.

RAM - Random Access Memory.

ROM - Read-Only Memory. A chip inside the modem which stores the factory default settings. This memory cannot be changed.

RTS - Request To Send

RX - Reception

S Register - RAM locations in the modem which store the active configuration.

Serial Port - See COMx.

TCM - Trellis-Coded Modulation

Touchtone Dialin - A dialing format in which each digit is represented by a musical frequency.

TX - Transmission

8-ASCII CODE TABLE

Decimal	Hex	Value	Decimal	Hex	Value	Decimal	Hex	Value	Decimal	Hex	Value

000	00	NUL	032	20	(space)	064	40	@	096	60	'

001	01	SOH	033	21	!	065	41	A	097	61	a

002	02	STX	034	22	"	066	42	B	098	62	b

003	03	ETX	035	23	#	067	43	C	099	63	c

004	04	EOT	036	24	$	068	44	D	100	64	d

005	05	ENQ	037	25	%	069	45	E	101	65	e

006	06	ACK	038	26	&	070	46	F	102	66	f

007	07	BEL	039	27	'	071	47	G	103	67	g

008	08	BS	040	28	(	072	48	H	104	68	h

009	09	HT	041	29	)	073	49	I	105	69	i

010	0A	LF	042	2A	*	074	4A	J	106	6A	j

011	0B	VT	043	2B	+	075	4B	K	107	6B	k

012	0C	FF	044	2C	,	076	4C	L	108	6C	l

013	0D	CR	045	2D	-	077	4D	M	109	6D	m

014	0E	SO	046	2E	.	078	4E	N	110	6E	n

015	0F	SI	047	2F	/	079	4F	O	111	6F	o

016	10	DLE	048	30	0	080	50	P	112	70	p

017	11	DC1	049	31	1	081	51	Q	113	71	q

018	12	DC2	050	32	2	082	52	R	114	72	r

019	13	DC3	051	33	3	083	53	S	115	73	s

020	14	DC4	052	34	4	084	54	T	116	74	t

021	15	NAK	053	35	5	085	55	U	117	75	u

022	16	SYN	054	36	6	086	56	V	118	76	v

023	17	ETB	055	37	7	087	57	W	119	77	w

024	18	CAN	056	38	8	088	58	X	120	78	x

025	19	EM	057	39	9	089	59	Y	121	79	y

026	1A	SUB	058	3A	:	090	5A	Z	122	7A	z

027	1B	ESC	059	3B	;	091	5B	[	123	7B	{

028	1C	FS	060	3C	<	092	5C	\	124	7C	|

029	1D	GS	061	3D	=	093	5D	]	125	7D	}

030	1E	RS	062	3E	>	094	5E	^	126	7E	~

031	1F	US	063	3F	?	095	5F	_	127	7F	DEL