Instructions for the

HUG Both Sides Printer Utility

The  Both Sides Printer Utility is a program that lets you  print

disk  text files that are formatted into pages on both  sides  of

the  paper.  With Both Sides, you can print all of those  "share-

ware",  public domain, and HUG program .DOC files using half  the

amount  of paper and half the notebook space that you would  nor-

mally  have to use.  Both Sides works by printing all of the  odd

pages  (1, 3, etc.) first.  Then it stops and allows you  to  re-

load  your paper with the other side facing the print  head,  and

then  it prints the even pages (2, 4, etc.).  Both Sides is  very

easy  to use, and can print several files with one  command.   If

you have a lot of .DOC files to print, it will pay for itself  in

saved paper and notebook space.

Requirements

Both  Sides will run on any MS-DOS or PC-DOS based computer  (in-

cluding  the Z-100) with at least 32k free memory and  MS-DOS  or

PC-DOS version 2 or above.  You will, of course, need a  printer,

and  you  should use fan-fold paper that can be run  through  the

printer twice and printed on both sides.

Getting Started

To  use  Both  Sides,  you need to  copy  the  files  BS.COM  and

REPRINT.COM from this disk to your system disk or partition.  You

should  copy  them to either the root directory, or  a  directory

pointed  to by your PATH description.  If you have a  Z-100,  you

may also want to copy the file WRAP.COM from this disk.

Here  is the procedure for copying the files if you have  a  hard

disk system.

1.   Make the drive and directory where you want to place  BS.COM

and  REPRINT.COM (and WRAP.COM if you have a Z-100) the  de-

fault drive and directory.

2.   Place  the HUG Both Sides Printer Utility disk in drive  A:,

and enter:

COPY A:BS.COM

COPY A:REPRINT.COM

COPY A:WRAP.COM        (enter this if you have a Z-100.)

Here is the procedure for copying the files if you have a floppy-

based system.

1.   Ensure that your system disk is in drive A:, and make A: the

default  drive.  If you want the Both Sides programs to  re-

side in a directory other than the root, make it the default

directory.

1

2.   If  you  have two drives, place the HUG Both  Sides  Printer

Utility disk in drive B:.

3.   Enter these commands:

COPY B:BS.COM

COPY B:REPRINT.COM

COPY B:WRAP.COM        (enter this if you have a Z-100.)

(If you do not have a B: drive, MS-DOS will instruct you  to

place the B disk (the HUG disk) in your A: drive when it  is

needed.

Using the Both Sides Utility

When  the  Both Sides utility is properly copied to  your  system

disk, you can use it to print files by entering a command in this

syntax:

BS pathname[,pathname,etc..][/Ln][/On][/2]

where  pathname is the path description (drive and  directory  if

needed,  and name) of each file you want to print.  The  optional

switches (introduced by the / character) are:

/Ln  Set  the page length to n lines, where n is a number from  1

to  99.   The default, if you do not use the switch,  is  66

lines.   This setting only governs pagination if line  feeds

only are used to advance the paper.  If form feed characters

are  used  in your files, the form length  setting  of  your

printer will govern page length.

/On  Set  the offset of each line to n spaces from the left  mar-

gin, where n is a number from 0 to 99.  This setting  allows

you  to provide even margins on each side of the text.   The

default  is  8 spaces.  You should use the TYPE  command  to

view each document before you print it, to determine if  the

lines of text area already offset from the left margin.   If

they  are,  you may want to use /O0 in the command  line  to

suppress the addition of more offset.

/2   Print  the "second side" (pages 2, 4, 6, etc.) only  of  the

specified file(s).  This option is useful when you are using

the  Both  Sides utility with a background spooler  such  as

ZSPOOL or HUGBPS, as will be explained later.

Here is an example of how to use the Both Sides utility.  Suppose

you have three files, called CHAPTER.1, CHAPTER.2, and CHAPTER.3.

These files were produced by having WordStar(tm) write a file  to

disk,  so the lines of text are already offset 8 characters  from

the left margin.  The command to print these files is:

BS CHAPTER.1,CHAPTER.2,CHAPTER.3/O0

Notice  that we have set the offset to 0. After this  command  is

entered, the Both Sides utility will sign on and then print

2

Printing side 1 of CHAPTER.1.

It  will  print similar messages as it prints  the  other  files.

Then it will print the following message:

Side 1 printing completed.

Re-load your paper with the back side facing

the print head, and the top of the first

printed page aligned at the print head, and

press Return...

When you see this message, you should first wait for your printer

to  stop.   If you printer has some kind of memory  buffer,  this

message  may appear some time before printing has stopped.   When

printing stops, tear the ejected paper off at the fold closest to

the  printer.  Then feed the paper back into the printer so  that

the  side with printing on it faces away from the printhead,  and

align  the  top of the first printed page with  the  print  head.

Both  Sides will have ejected a blank sheet before  printing,  so

make sure that printing will start on the back side of the  first

printed page.  Both Sides will not eject a page first during this

pass.  When the paper is ready, press Return, and Both Sides will

begin printing again, and will display

Printing side 2 of CHAPTER.1.

and similar messages, until printing is completed.

Using a Spooler or Print Buffer

If you use a background spooler such as ZSPOOL or HUGBPS, or  you

have  a print buffer, Both Sides will prompt you to re-load  your

paper  long before printing is completed.  If you would  like  to

use your computer during this period, rather than wait for print-

ing to stop, you can type Control-Break (PC-compatibles only)  or

Control-C  when you see the "Re-load your paper"  message.   Both

Sides will exit to DOS, allowing you to use your computer.   When

printing  stops, re-load your paper as described above,  and  run

Both sides again using the identical command line you did for the

first  part of the document, but add the /2 switch.  For our  ex-

ample above, the command would be:

BS CHAPTER.1,CHAPTER.2,CHAPTER.3/O0/2

Don't forget to include any switches that you used in the command

line  for  the first pass.  Remember, the command lines  must  be

identical  except for the addition of the /2 switch.  Both  Sides

will print the second side of the document(s) the same as if  you

had pressed Return at the "Re-load" prompt.

When  you are figuring how much spooler or buffer memory will  be

required to print a list of files, remember that Both Sides  only

prints  half  of a file during each pass.  Therefore  you  should

divide  the  total of the sizes of the files in half to  get  the

spooler or buffer memory required.

3

Page Numbering

When we say that Both Sides prints the odd numbered pages on  one

side  of the paper, and the even numbered pages on the other,  we

are  referring to the way Both Sides numbers the pages,  and  not

necessarily to the actual page numbering.  For example, suppose a

document  has  an un-numbered title page followed by  text  pages

numbered in sequence (1, 2, etc.).  Both Sides would consider the

title  page  as page 1, and the first numbered page (page  1)  as

page 2.  It would therefore print the title page and pages 2,  4,

etc. on the "front" side of the paper, and then print pages 1, 3,

etc. on the "back" side.

Some  documents  have separate page numbering for  each  chapter,

with  all  of the chapters in a single file.  In this  case,  you

might have a mixture of odd and even numbered pages on each  side

of the paper.  Just remember that Both Sides numbers the pages by

actual count and not by any page numbers that happen to be on the

pages.  It does its best to ensure that things come out right, so

that the proper pages are opposite each other on each sheet.

Error Messages

If  something goes wrong when you run the Both Sides utility,  it

may print one of the following error messages.

"This program requires MS-DOS version 2 or above."  You will  see

this  message if you attempt to run Both Sides on a  DOS  version

less than 2.

"Not  enough  free memory to run this program."  Both  Sides  re-

quires  at least 32k of free memory space, and will display  this

message if it does not see enough.

"Can't find <pathname>."  This message will be displayed (with an

actual path name instead of <pathname>) if Both Sides cannot  lo-

cate  one  of the files specified on the command line.   It  will

skip that file and go on to the next one in the list, if any.  If

you  are printing the first side of the document(s), and you  in-

tend  to Control-Break or Control-C out of the program  when  the

"Re-load"  prompt appears, you should leave out the skipped  path

name  when  you re-type the command line.  If the  file  was  not

found  because you typed the name wrong and you happened to  type

it  correctly the second time, your document pages would be  "out

of sync", with the wrong pages on opposite sides of the paper.

"Error  reading <pathname>."  If Both Sides has  trouble  reading

one of the specified files, it will print this message, with  the

actual  path  name instead of <pathname>.  It will then  exit  to

DOS.  If this happens during the printing of side one of  several

documents, you can still print side two of the successfully prin-

ted  ones by repeating the command line with all files up to  the

bad one, and using the /2 switch.

Note:   If  you run Both Sides without specifying a file  in  the

command line, it will display brief instructions on using it.

4

The Reprint Utility

The  Reprint Utility allows you to re-direct to a disk  file  the

output  of  a program that would normally go to a  printer.   The

resulting file can later be printed using Both Sides or any other

method for printing a disk file.  To use Reprint, enter a command

line using this syntax:

REPRINT <output file> <program> [<arguments>]

where <output file> is the name of the file where printer  output

is  to be placed, <program> is a path description of the  program

you  want to run, and <arguments> are any command line  arguments

required by the program.  For example, suppose you want to run  a

BASIC program called TESTPROG.BAS that prints, and re-direct  its

printer  output.  The normal command you would use might be  like

this:

BASICA TESTPROG

To  run the program with Reprint, and re-direct its output  to  a

file called FILE.PRN, you would enter:

REPRINT FILE.PRN BASICA TESTPROG

This  command  line assumes that BASICA and TESTPROG are  in  the

current  directory,  and  that the file FILE.PRN is  also  to  be

placed  in the current directory.  If BASICA was in  a  directory

called BIN, TESTPROG was in a directory called BAS, and  FILE.PRN

was  to  be placed in a directory called TEXT, the  command  line

would be:

REPRINT \TEXT\FILE.PRN \BIN\BASICA \BAS\TESTPROG

Note that even if your path description was set up so that BASICA

could  normally  be run without specifying the  path,  you  would

still have to specify it in the Reprint command line.

Reprint  runs  the program you specify in the command line  as  a

"child"  program, and re-directs any printer output attempted  by

that program into the file you specify.  When your program exits,

Reprint closes the print file and also exits.

Printing the Reprint File

To  print the file produced by Reprint, you can use the DOS  COPY

command to copy the file to your printer.  For example, to  print

FILE.PRN, you could use this command:

COPY FILE.PRN PRN

You can also use Both Sides to print the file, if it is  properly

divided into pages.

The file produced by Reprint contains all of the data that  would

have  been  sent to your printer, including any  special  printer

codes the program may have produced.  If the program is set up to

5

support  a special type of printer, the file may produce  unsuit-

able  results if copied to another type of printer.  If the  pro-

gram  is set up to support a Diablo or compatible printer, or  if

it  is set up to drive the printer in a graphics mode,  the  file

produced  by Reprint will be considerably larger than the  number

of bytes used by the actual text.

If you are planning to make a print file by using Reprint to  run

a  word  processing program, and then print the  file  with  Both

Sides,  you  may want to set up your word processing  program  so

that the specified printer type is a "draft printer" rather  than

your specific printer type.  The special codes for some  printers

may  affect Both Sides' ability to paginate correctly.   This  is

especially  the  case if your printer is a Diablo  or  compatible

printer, or if it is driven in a graphics mode by your word  pro-

cessor.   If your word processor is able to produce several  dif-

ferent  fonts or type sizes on your printer, then it is  probably

driving it in a graphics mode.

The  "draft printer" driver of your word processor  will  produce

acceptable  results with most printers, and it will  not  produce

any special codes that can adversely affect Both Sides.

Reprint Error Messages

If something goes wrong when you run Reprint, you may see one  of

these error messages.

"This program requires MS-DOS version 2 or above."  You will  see

this message if you attempt to run Reprint on a DOS version  less

than 2.

"Can't find <program>" where <program> is the name of the program

you  are trying to run.  You will see this message if  you  typed

the program name incorrectly, or if you did not specify the prop-

er path so that Reprint can find it.

"Can't execute <program>" where <program> is the name of the pro-

gram  you  are trying to run.  You will see this message  if  for

some reason Reprint is unable to execute your program as a  child

under  itself.  One possible cause would be not enough memory  to

support both Reprint and your program.

"Unable  to write <output file>" where <output file> is the  file

where  printer  output is to go.  You will get  this  message  if

Reprint is unable to create or write the output file.  Note  that

if  there is not enough disk space for the output  file,  Reprint

may  produce  a truncated file rather than  displaying  an  error

message.

If  you do not specify enough information in the Reprint  command

line, it will just print some brief instructions and exit.

6