%OP%DESystem Calls Reference

%OP%BON

%OP%AMM

%OP%HM3

%OP%FM3

%OP%BM0

%OP%LM10

%OP%HE/@T@

%OP%FO//Z88 Developer's Notes V3, @P@//

%CO:A,5,74%OS_PRT, send character directly to printer filter

RST 20H, DEFB $24

IN:

OUT, if call successful:

OUT, if call failed:

Registers changed after return:

Notes:

Bugs:

%P0%

%CO:B,5,69%

A = character to be written to printer filter

Fc = 0

Fc = 1

A = error code:

..BCDEHL/IXIY same

AF....../.... different

It is generally preferred to open a channel to the printer filter

and use the usual routines.

%CO:C,5,64%

RC_ESC ($01), escape detected

RC_WP, write protected

%CO:D,5,59%%CO:E,5,54%%CO:F,5,49%